## Appendix 6C-1	IDF Curves with B, D, and E Factors

### B, D and E Factors that Define Intensity-Duration-Frequency (IDF) Curves

### for Use with the Rational Method and the Modified Rational Method

The rainfall IDF curves are also described by the equation:

Where:

i	=   Intensity, inches per hour (in/hr)

tc	=   Time of concentration, minutes (min)

The B, D and E factors are available for individual counties and may be obtained through the VDOT Central Office Hydraulics Section.

A sample of the information available is shown in Appendix 6C-2.

### Source:

Regression Constants from Virginia Stormwater Management Handbook, 1st Ed., Vol. II, 1999

## Appendix 6C-1	IDF Curves with B, D, and E Factors

### Rainfall Report – Chesterfield County

#### Rainfall Type:		Custom Rainfall

Rainfall Filename:	Chesterf.rnd

INTERMEDIATE INTENSITIES (in/hr)

5 min		15 min	30 min	60 min	6 hr		24 hr

2 yr	 5.71		 3.80		  2.56		   1.62		0.40		0.13

5 yr	 6.56		 4.60		  3.16		   2.02	 	0.48		0.14

10 yr	 7.22		 5.16		  3.58		   2.31		0.54		0.16

25 yr	 8.21	 	 6.00		  4.20		   2.72		0.66		0.19

50 yr	 8.99		 6.68		  4.68		   3.03		0.67		0.18

100 yr	 9.77		 7.32		  5.14		   3.35		0.78		0.22

BDE VALUES 	Intensity =

B		  D		  E

2 yr		58.01		10.75		0.84

5 yr		92.96		14.75		0.89

10 yr	          111.70		16.25		0.90

25 yr	          146.51		18.25		0.91

50 yr	          213.71		21.25		0.97

100 yr	          221.97		21.25		0.95