## Appendix 6G-3    	Alternate Values for Coefficient of

## Runoff in Urban Areas

| #### Description of Area | Runoff Coefficients | Percent Impervious |
| --- | --- | --- |
| Business, Commercial & Industrial | 0.80-0.90 | 90% |
| ##### Apartments & Townhouses | 0.65-0.75 | 75% |
| Schools & Churches | 0.50-0.60 | 50% |
| Residential: |  |  |
| Single Family Units |  |  |
| ##### Lots 10,000 SF | 0.40-0.50 | 35% |
| Lots, 12,000 SF | 0.40-0.45 | 30% |
| Lots, 17,000 SF | 0.35-0.45 | 25% |
| Lots ½ Acre or More | 0.30-0.40 | 20% |
| Parks, Cemeteries & Unimproved Areas | 0.25-0.35 | 15% |
| Pavement & Roofs | 0.90 | - |
| Lawns | 0.25-0.35 | varies |

Source: