## Appendix 7B-4			RIPRAP DESIGN WORK SHEET

## For Standard VDOT Riprap Sizes Only

CHANNEL DATA

Q = _________(cfs) 		P = _________(ft.) 		n = ________

So = _________(ft/ft)     	R = _________(ft.)

dn = _________(ft.)      	Vn = _________(fps)

A = _________(ft.2)   		Side Slope = _____ :1

#### DETERMINE RIPRAP SIZE

 = 42o   Side Slope = _____ :1     = ______o

K1 = [1 - (sin2  / sin2 )] 0.5

K1 = [1 - (sin2 _______o / sin2 42o)] 0.5 = ________

For Specific Gravity  = 2.65 and Stability Factor = 1.2

D50 = 0.001  Va3 / (davg 0.5  K1 1.5)

D50 = 0.001  ________3 / ( ________0.5  ________1.5)

D50 Computed =  _____________

Note: All VDOT standard riprap (Class AI through Type II) is assumed to have a  of approximately 42o and a Specific Gravity of 2.65. Therefore, the Computed D50 should be adjusted by the Stability Correction Factor (CSF) (if any) to derive a Final D50. The VDOT standard class of riprap with the next higher D50 should be specified.

Correction Factor For Stability Factor (SF) other than 1.2 (Default = 1.0)

CSF = (SF / 1.2)1.5 = ( ________ / 1.2)1.5 = ________

Final D50 = CSF  Computed D50 = ________  ________ = ________

RIPRAP RECOMMENDATION: VDOT (Class) (Type) ____________

Thickness (T ) = _________” (2  D50 MSD minimum)

Source:	VDOT

Comments and/or Source