# Appendix 7D-6	Permissible Velocities

# for Erodible Linings

Permissible velocities for channels with erodible linings, based on uniform flow in continuously wet, aged channels1:

| Soil type or lining (earth; no vegetation) | ### Maximum permissible velocities for |  |  |
| --- | --- | --- | --- |
|  | Clear water | #### Water carrying fine silts | ### Water carrying sand and gravel |
| Fine sand (noncolloidal) Sandy loam (noncolloidal) Silt loam (noncolloidal) Ordinary firm loam Volcanic ash Fine gravel Stiff clay (very colloidal) Graded, loam to cobbles (noncolloidal) Graded, silt to cobbles (colloidal) Alluvial silts (noncolloidal) Alluvial silts (colloidal) Coarse gravel (noncolloidal) Cobbles and shingles Shales and hard pans | F.p.s. 1.5 1.7 2.0 2.5 2.5 2.5 3.7 3.7 4.0 2.0 3.7 4.0 5.0 6.0 | F.p.s. 2.5 2.5 3.0 3.5 3.5 5.0 5.0 5.0 5.5 3.5 5.0 6.0 5.5 6.0 | F.p.s. 1.5 2.0 2.0 2.2 2.7 3.7 3.0 5.0 5.0 2.0 3.0 6.5 6.5 5.0 |

Source:

1As recommended by Special Committee on Irrigation Research, American Society of Civil Engineers, 1926.