# Appendix   9A-1 	 Definitions and Abbreviations

Definitions:

Check Storm	The use of a less frequent event, such as a 50-year storm, to assess hazards at critical locations where water can pond to appreciable depths is commonly referred to as a check storm or check event.

Hydraulic Grade Line	The elevation to which the water can be expected to rise within a storm drain (pressure head +elevation head)

Spread	The width of flow measured laterally from the flowline.  With a curbed only section of roadway, the flowline is formed by the intersection of the pavement to the curb.  With a curb and gutter section, it is the intersection of the gutter pan and the curb.

Storm Drain	A storm drain system is a drainage system installed to carry stormwater runoff, consisting of two or more pipes in a series connected by one or more drop inlets.  An exception to this general rule is:  one or more cross drain pipes connected by one or more drop inlets, “hydraulically designed” to function as a culvert(s) and not connected to a storm drain system.

Velocity Head	A quantity of energy head proportional to kinetic energy of flowing water.

Abbreviations:

FEMA				Federal Emergency Management Agency

FHWA			Federal Highway Administration

NRCS			National Resources Conservation Service

VDOT					Virginia Department of Transportation