###### Appendix 14B-1	Checklist

CHECKLIST FOR PLANS SUBMITTED TO VDOT FOR APPROVAL

Yes	No	I. Plans

___	___	1.	North arrow shown on each sheet

___	___	2.	Centerline, __ __ curve data, __ __ stations, __ __  lot lines

& numbers shown.

___	___	3. 	R/W lines including dimensions (radius or radial lines)

shown.

___	___	4. 	Slope easements for fill slopes where slopes exceed

proposed right of way limits shown.

___	___	5. 	Names of adjacent subdivisions and/or property owners

denoted.

___	___	6. 	Subdivision limits shown.

___	___	7. 	Existing state route numbers and subdivision street names

shown.

___	___	8. 	Approved or proposed street names shown.

___	___	9. 	Destination of routes and streets shown.

___	___	10.	Radii for easements, turn-arounds, cul-de-sac, flares, etc.,

shown.

___	___	11.	Guardrail where needed shown.

___	___	12. 	Vicinity map shown with location referenced 0.01 mile to

nearest intersection.

___	___	13. 	General notes satisfactory. (Also see sheet 6 of 6)

___	___	14. 	Location of paved ditches by station shown.

___	___	15. 	Entrance pipe sizes shown for each lot.

___	___	16. 	Proposed drainage layout and description shown.

___	___	17. 	Erosion control stone and type shown.

___	___	18. 	Channel change and outlet ditches and easements shown.

___	___	19. 	Zoning shown.

___	___	20. 	Projected traffic count for each street between intersections

shown.

###### Appendix 14B-1	Checklist

Yes	No 	II. PROFILE & GRADE

___	___	1. 	Existing ground line shown.

___	___	2. 	Grade line (mainline and connections) shown.

___	___		a.	% of grade, CG, VC, and VSD shown.

___	___		b.	Finished grade elevations (50’ on tangent, 25’ on vertical

curve).

___	___		c. 	Station on profile agrees with plan stations.

___	___		d.	Name of streets shown.

___	___	3. 	Invert elevations on proposed drainage structures shown.

___	___	4. 	Profile and grades for outfall ditches shown.

___	___	5.  	Grades for special channels and ditches shown.

___	___	6. 	Profile for proposed storm sewers shown. (hydraulic grade

line to be introduced where applicable).

___	___	7. 	Sight distance at intersection and entrance satisfactory.

Yes	No 	III. GRADING PLAN

___	___	1.	Existing contours shown.

___	___	2.	Proposed grading contours shown.

___	___	3.	Contour of computed headwater shown.

___	___	4.	Foundation elevations of existing dwellings shown if

questionable.

___	___	5.	Design layout for drainage system shown.

Yes	No 	IV. TYPICAL SECTIONS - Geometrics shall be governed by traffic counts. (Minimum TC = no. of lots x 7.5)

___	___	1. 	Projected traffic count shown for each street between

intersections shown.

___	___	2. 	Centerline shown.

___	___	3. 	Point of finished grade noted.

___	___	4.	Surface *	Base *	Subbase*

__	__       		a. width            	__	__	a. width	__	__	a. width

__	__		b. cross slope	__	__	b. cross slope	__	__	b. cross slope

__	__      		c. type		__	__	c. type	__	__	c. type

__	__       		d. depth		__	__	d. depth	__	__	d. depth

###### Appendix 14B-1	Checklist

(* According to secondary roads current criteria.)

___	___	5. Shoulders	Ditches	Slopes

__	__		a. cut		__	__	a. width	__	__	a. ratio

__	__		b. fill		__	__	b. depth	__	__	b. rounding

__	__		c. slope ratio	__	__	c. slope ratio	__	__	c. seeding

Same as above for each different road typical section required.

___	___	6. 	Same as above for each different road typical section

required.

___	___	7. 	Channel changes, outlet and inlet ditches shown.

___	___		a. bottom width	___	___		d. length

___	___		b. side slopes	___	___		e. If lined, show depth and

___	___		c. depth				    type of lining

___	___	8. 	Special design typical sections included.

___	___	9. 	Stations to which typical sections apply shown under

typical section.

Yes	No	V. DRAINAGE GENERAL

___	___	1. 	Existing or proposed drainage easements shown to water

course.

___	___	2. 	Direction of flow by arrows for pipe and ditches shown.

___	___	3. 	Drainage areas in acres (Supported by outlined contour

map).

___	___	4. 	Pipe sizes to be in accordance with VDOT standard sizes.

___	___	5. 	Items not VDOT standard are to be detailed.

___	___	6. 	Type of pipe to be installed (on and off right of way noted).

___	___		a. Design cover shown

___	___	7. 	Design computations computed according to current VDOT

criteria included.

Yes	No	VI. DRAINAGE DESIGN

1.	Hydrologic breakdown.

___	___		a.	Reference to material used if not VDOT

b. 	Rational method: 	Q = CIA

___	___		  	1.	“C” – 	breakdown of planned development and

related areas shown.

###### Appendix 14B-1	Checklist

___	___			2.	“I” – 	(a)	overland flow (slope, type cover included)

(b) 	channel flow (typical section lining, slope,

and included velocity)

___	___			3.	“A” – 	DA in acres (supported by outlined contour map)

___	___		c.	Other methods – (supported by factors used in

computation of Q)

2.	Hydraulic breakdown

a.	Open channels (capacity)

___	___		1.	Computation to support design included.

___	___		2.	Capacity (lined, unlined, and combination lining)

___	___			(a)	depth

___	___			(b)	type of lining

___	___			(c)	velocity of flow

b. 	Open channels (erosion control)

___	___			1. 	Computation supporting lined ditches included

(See also sheet 6 of 6)

c. 	Curb and gutter (computation to support the following):

___	___			1.	Depth and Spread of water in gutter.

___	___			2.	Length of slots and placements of inlets.

___	___			3.	Type of inlet proposed.

d. 	Storm sewers (computation to support the following) to

include:

___	___			1.	Velocity of flow.

___	___			2.	Minimum velocity of 3 fps maintained.

___	___			3.	Elevation of the hydraulic grade line at key points 								(drop inlets, manholes, etc.) noted.

___	___			4. 	Size of pipe.

e. 	Special design structures (computations to support the

following include):

___	___			1.	Special design entrance for culverts.

___	___			2.	Flumes (Q-dn, vn, freeboard)

___	___			3.	Energy dissipators.

___	___			4.	Special channels (curve, mitered, channel confluences, etc.)

f. 	Culverts

___	___	1.	Invert	___	___	6.	Outlet velocity

___	___	2. 	Length	___	___	7.	Erosion control &

___	___	3.	Type	___	___		type (where outlet

___	___	4.	Headwater				velocity exceeds 6 fps)

depth	___	___ 	8.	Size

___	___	5.	End    	___	___

treatment

g.	Flood control structures

___	___	  		1.	When proposed, consult District Drainage Engineer

Yes	No	VII. DRAINAGE OUTFALLS

___	___	1.	Has been field checked for location and found satisfactory?

___	___	2.	Is the length of easement shown satisfactory?

###### Appendix 14B-1	Checklist

___	___	3.	Is width of easement shown satisfactory for maintenance?

___	___	4.	Is easement shown on plat?

___	___	5.	Is erosion a problem?

___	___		a.	Will this development create an erosion problem?

___	___	6.	Is siltation now a problem?

___	___		a.	Will this development create a siltation problem?

___	___	7.	Are there obstructions in or is downstream maintained by

the department?

___	___		a.	Are there existing problems?

___	___		b.	Will this development cause problems?

___	___	8.	Is there drainage being diverted to this outfall?

___	___	9.	Are there other developments (future or proposed) which affect

outfall?

___	___	10.	Are there any proposed road projects which could adversely

affect or be affected by this proposed development?

Yes	No	VIII. GENERAL NOTES

___	___	A..	Are the following notes included as a part of the plans?

If not, should they be included?

Which ones? ____________________________________________

- VDOT approval of subdivision road plans does not preclude the right to add additional facilities such as seeding, paving, sediment control items, etc., as may be deemed necessary by the Department prior to the acceptance of such roads in order to limit siltation and pollution of nearby lakes, ponds, streams, and adjacent property.

- Are provisions included in plans to protect adjacent property from siltation?

2.	The Department’s approval of these plans expires three years from data approval.

Yes	No	IX. DESIGN CRITERIA

A.	Drainage Design

___	___		1.	Are spans, culverts, storms sewers, and channel capacity

being computed on a minimum 10-year flood frequency?

___	___		2.	Are the size of curb drop inlet slots being computed with

the proper rainfall?

###### Appendix 14B-1	Checklist

___	___		3.	Is the following policy being followed in computing and

proposing paved ditches:

a.	Ditches shall be paved where computed velocities exceed the appropriate allowable velocity unless supporting soils or channel lining analysis indicate greater velocities can be tolerated.

b.	Ditches shall be paved when conditions and unusual situations are encountered and/or directed by the Highway Engineer in writing.

c.	Additional paved ditches shall be provided where necessary to meet field conditions to prevent erosion as directed by the engineer in writing.

d.	Paved ditches may be deleted as deemed necessary as directed by the engineer in writing.