General Permit No.: DCR01

Effective Date: July 1, 2004

Expiration Date: June 30, 2009

GENERAL PERMIT FOR DISCHARGES OF STORMWATER FROM

CONSTRUCTION ACTIVITIES

AUTHORIZATION TO DISCHARGE UNDER THE VIRGINIA STORMWATER

MANAGEMENT PROGRAM AND THE VIRGINIA STORMWATER

MANAGEMENT ACT

In compliance with the provisions of the Clean Water Act, as amended, and pursuant to

the Virginia Stormwater Management Act and regulations adopted pursuant to that,

operators of construction activities (those sites or common plans of development or sale

that will result in the disturbance of one or more acres of total land area) with stormwater

discharges from these construction activities are authorized to discharge to surface waters

within the boundaries of the Commonwealth of Virginia, except those specifically named

in State Water Control Board and Virginia Soil and Water Conservation Board

regulations and policies or permit issuing authority policies and ordinances which

prohibit such discharges.

The authorized discharge shall be in accordance with this cover page, Section I -

Discharge Authorization and Special Conditions, Section II - Stormwater Pollution

Prevention Plan, and Section III - Conditions Applicable to All VSMP Permits as set

forth herein.

SECTION I

DISCHARGE AUTHORIZATION AND SPECIAL CONDITIONS

A. Coverage under this permit.

1. During the period beginning with the date of coverage under this general permit and

lasting until the permit's expiration date, the permittee is authorized to discharge

stormwater from construction activities.

2. This permit also authorizes stormwater discharges from off-site support activities (e.g.,

concrete or asphalt batch plants, equipment staging yards, material storage areas,

excavated material disposal areas, borrow areas) provided that:

a. The support activity is directly related to a construction site that is required to have

VSMP permit coverage for discharges of stormwater associated with construction

activity;

b. The support activity is not a commercial operation serving multiple unrelated

construction projects by different operators, and does not operate beyond the completion

of the construction activity at the last construction project it supports; and

c. Appropriate controls and pollution prevention measures for the discharges from the

support activity areas are identified in the stormwater pollution prevention plan required

for the construction activity under Section II D of this permit.

3. There shall be no discharge of floating solids or visible foam in other than trace

amounts.

B. Limitation on coverage.

1. Post-construction discharges. This permit does not authorize stormwater discharges

that originate from the site after construction activities have been completed and the site,

including any temporary support activity site, has undergone final stabilization. Postconstruction industrial stormwater discharges may need to be covered by a separate

VPDES permit.

2. Discharges mixed with nonstormwater. This permit does not authorize discharges that

are mixed with sources of nonstormwater, other than those discharges which are

identified in Section I D 2 (Exceptions to prohibition of nonstormwater discharges) and

are in compliance with Section II D 5 (Nonstormwater discharge management).

3. Discharges covered by another permit. This permit does not authorize stormwater

discharges associated with construction activity that have been covered under an

individual permit or required to obtain coverage under an alternative general permit in

accordance with Part Section III X.

4. TMDL limitation. Discharges to waters for which a "total maximum daily load"

(TMDL) allocation for sediment or a parameter that addresses sediment (such as total

suspended solids, turbidity, or siltation) has been established by the State Water Control

Board and approved by EPA are not eligible for coverage under this permit unless the

stormwater pollution prevention plan (SWPPP) developed by the operator incorporates

measures and controls that are consistent with the assumptions and requirements of such

TMDL. To be eligible for coverage under this general permit, the SWPPP must

incorporate any conditions applicable to discharges from the construction site that are

necessary for consistency with the assumptions and requirements of the TMDL. If a

specific wasteload allocation has been established that would apply to discharges from

the construction site, the operator must incorporate that allocation into the SWPPP and

implement necessary steps to meet that allocation.

C. Commingled discharges. Any discharge authorized by a different VSMP or VPDES

permit may be commingled with discharges authorized by this permit.

D. Prohibition of nonstormwater discharges.

1. Except as provided in Sections I A 2, I C and I D 2, all discharges covered by this

permit shall be composed entirely of stormwater associated with construction activity.

2. The following nonstormwater discharges from active construction sites are authorized

by this permit provided the nonstormwater component of the discharge is in compliance

with Section II D 5 (Nonstormwater discharges):

a. Discharges from fire fighting activities;

b. Fire hydrant flushings;

c. Waters used to wash vehicles where detergents are not used;

d. Water used to control dust;

e. Potable water sources, including waterline flushings;

f. Water used for hydrostatic testing of new pipeline construction;

g. Routine external building wash down which does not use detergents;

h. Pavement washwaters where spills or leaks of toxic or hazardous materials have not

occurred (unless all spilled material has been removed) and where detergents are not

used;

i. Uncontaminated air conditioning or compressor condensate;

j. Uncontaminated ground water or spring water;

k. Foundation or footing drains where flows are not contaminated with process materials

such as solvents;

l. Uncontaminated excavation dewatering, and

m. Landscape irrigation.

E. Releases of hazardous substances or oil in excess of reportable quantities.

The discharge of hazardous substances or oil in the stormwater discharges from the

construction site shall be prevented or minimized in accordance with the stormwater

pollution prevention plan for the site. This permit does not relieve the permittee of the

reporting requirements of 40 CFR Part 110 (2002), 40 CFR Part 117 (2002) and 40 CFR

Part 302 (2002) or § 62.1-44.34:19 of the Code of Virginia.

Where a release containing a hazardous substance or oil in an amount equal to or in

excess of a reportable quantity established under either 40 CFR Part 110 (2002), 40 CFR

Part 117 (2002) or 40 CFR Part 302 (2002) occurs during a 24-hour period:

1. The permittee is required to notify the Department of Environmental Quality and the

permit issuing authority in accordance with the requirements of Section III G as soon as

he has knowledge of the discharge;

2. Where a release enters a municipal separate storm sewer system (MS4), the permittee

shall also notify the owner of the MS4 and the Department of Conservation and

Recreation; and

3. The stormwater pollution prevention plan required under Section II D of this permit

must be reviewed to identify measures to prevent the reoccurrence of such releases and to

respond to such releases, and the plan must be modified where appropriate.

F. Spills.

This permit does not authorize the discharge of hazardous substances or oil resulting

from an on-site spill.

G. Termination of permit coverage.

1. The operator of the construction activity may only submit a notice of termination after

one or more of the following conditions have been met:

a. Final stabilization has been achieved on all portions of the site for which the operator is

responsible;

b. Another operator has assumed control over all areas of the site that have not been

finally stabilized;

c. Coverage under an alternative VPDES or VSMP permit has been obtained; or

d. For residential construction only, temporary stabilization has been completed and the

residence has been transferred to the homeowner.

2. The notice of termination must be submitted within 30 days of one of the conditions in

Section I G 1 being met. Authorization to discharge terminates seven days after the

notice of termination is submitted.

3. The notice of termination shall be signed in accordance with Section III K of this

permit.

H. Water quality protection. The permittee must select, install, implement and maintain

best management practices (BMPs) at the construction site that minimize pollutants in the

discharge as necessary to meet applicable water quality standards. If there is evidence

indicating that the stormwater discharges authorized by this permit are causing, have the

reasonable potential to cause, or are contributing to an excursion above an applicable

water quality standard, or are causing downstream pollution (as defined in this part), the

permit issuing authority may take appropriate enforcement action, may require the

permittee to include and implement appropriate controls in the SWPPP to correct the

problem, and/or may require the permittee to obtain an individual permit in accordance

with 4VAC50-60-410 B 3.

SECTION II

STORMWATER POLLUTION PREVENTION PLAN

A stormwater pollution prevention plan (SWPPP) shall be developed and implemented

for the construction activity covered by this permit. SWPPPs shall be prepared in

accordance with good engineering practices. The SWPPP shall identify potential sources

of pollution which may reasonably be expected to affect the quality of stormwater

discharges from the construction site. In addition, the SWPPP shall describe and ensure

the implementation of practices which will be used to reduce pollutants in stormwater

discharges from the construction site, and to assure compliance with the terms and

conditions of this permit.

The SWPPP requirements of this general permit may be fulfilled by incorporating by

reference other state, tribal or local plans such as an erosion and sediment control (ESC)

plan, a spill prevention control and countermeasure (SPCC) plan developed for the site

under § 311 of the federal Clean Water Act or best management practices (BMP)

programs otherwise required for the facility provided that the incorporated plan meets or

exceeds the SWPPP requirements of Section II D. If an erosion and sediment control plan

for the construction activity is being incorporated by reference, the referenced plan must

be approved by the locality in which the construction activity is to occur or by another

appropriate plan approving authority authorized under the Erosion and Sediment Control

Regulations (4 VAC 50-30) prior to the commencement of construction. All plans

incorporated by reference into the SWPPP become enforceable under this permit. If a

plan incorporated by reference does not contain all of the required elements of the

SWPPP of Section II D, the permittee must develop the missing elements and include

them in the required SWPPP.

Once a definable area has been finally stabilized, the operator may mark this on the

SWPPP and no further SWPPP or inspection requirements apply to that portion of the site

(e.g., earth disturbing activities around one of three buildings in a complex are done and

the area is finally stabilized; one mile of a roadway or pipeline project is done and finally

stabilized, etc.).

The operator must implement the SWPPP as written from commencement of construction

activity until final stabilization is complete.

A. Deadlines for SWPPP preparation and compliance.

1. The SWPPP shall be prepared prior to submittal of the registration statement and

provide for compliance with the terms and schedule of the plan beginning with the

initiation of construction activities.

2. For ongoing construction activity involving a change of operator, the new operator

shall accept and maintain the existing SWPPP, or prepare and implement a new SWPPP

prior to taking over operations at the site.

B. Signature, plan review and making plans available.

1. The SWPPP shall be signed in accordance with Section III K.

2. The SWPPP shall be retained, along with a copy of this permit at the construction site

from the date of commencement of construction activity to the date of final stabilization.

Permittees with day-to-day operation control over SWPPP implementation shall have a

copy of the plan available at a central location on-site for the use of all operators and

those identified as having responsibilities under the plan whenever they are on the

construction site. The SWPPP must be made available, in its entirety, to the department

and the permitting issuing authority for review at the time of an on-site inspection.

3. The permittee shall make SWPPPs available upon request to the department; the

permit issuing authority; a state or local agency approving erosion and sediment plans,

grading plans, or stormwater management plans; local government officials; or the

operator of a municipal separate storm sewer system receiving discharges from the site.

C. Maintaining an updated SWPPP.

1. The permittee shall amend the SWPPP whenever there is a change in design,

construction, operation, or maintenance that has a significant effect on the discharge of

pollutants to surface waters and that has not been previously addressed in the SWPPP.

2. The SWPPP must be amended if during inspections or investigations by site staff, or

by local, state or federal officials, it is determined that the discharges are causing water

quality exceedances, or the SWPPP is ineffective in eliminating or significantly

minimizing pollutants in stormwater discharges from the construction site.

3. Based on the results of an inspection, the SWPPP must be modified as necessary to

include additional or modified BMPs designed to correct problems identified. Revisions

to the SWPPP must be completed within seven calendar days following the inspection.

Implementation of these additional or modified BMPs must be accomplished as described

in Section II D 3 b.

4. The SWPPP must clearly identify for each measure identified in the plan, the

contractor(s) or subcontractor(s) that will implement the measure. The SWPPP shall be

amended to identify any new contractor that will implement a measure of the plan.

D. Stormwater pollution prevention plan contents.

The SWPPP shall include the following items:

1. Site and activity description. Each SWPPP shall provide the following information:

a. A description of the nature of the construction activity, including the function of the

project (e.g., low density residential, shopping mall, highway, etc.);

b. The intended sequence and timing of activities that disturb soils at the site (e.g.,

grubbing, excavation, grading, utilities and infrastructure installation).

c. Estimates of the total area expected to be disturbed by excavation, grading, or other

construction activities including off-site borrow and fill areas;

d. A description of any other potential pollution sources, such as vehicle fueling, storage

of fertilizers or chemicals, sanitary waste facilities, etc.

e. Identification of the nearest receiving waters at or near the construction site that will

receive discharges from disturbed areas of the project;

f. The location and description on any discharge associated with industrial activity other

than construction at the site. This includes stormwater discharges from dedicated asphalt

plants and dedicated concrete plants that are covered by this permit.

g. A site map indicating:

(1) Directions of stormwater flow and approximate slopes anticipated after major grading

activities;

(2) Areas of soil disturbance and areas of the site which will not be disturbed;

(3) Locations of major structural and nonstructural controls identified in the SWPPP,

including those that will be permanent controls that will remain after construction

activities have been completed;

(4) Locations where stabilization practices are expected to occur;

(5) Surface water bodies (including wetlands);

(6) Locations where stormwater discharges to a surface water;

(7) Locations of off-site material, waste, borrow or equipment storage areas covered by

the plan;

(8) Locations of other potential pollution sources , such as vehicle fueling, storage of

chemicals, sanitary waste facilities, etc.; and

(9) Areas where final stabilization has been accomplished and no further construction-phase permit requirements apply.

2. Controls to reduce pollutants. The SWPPP shall include a description of all pollution

control measures that will be implemented as part of the construction activity to control

pollutants in stormwater discharges. For each major activity identified in the project

description, the SWPPP shall clearly describe appropriate control measures, the general

sequencing during the construction process in which the measures will be implemented,

and which operator is responsible for the control measure's implementation.

a. Erosion and sediment controls.

(1) Stabilization practices. The SWPPP shall include a description of interim and

permanent stabilization practices for the site. Site plans should ensure that existing

vegetation is preserved where attainable and that disturbed portions of the site are

stabilized. Stabilization practices may include, but are not limited to: temporary seeding,

permanent seeding, mulching, geotextiles, sod stabilization, vegetative buffer strips,

protection of trees, preservation of mature vegetation, riprap, gabions, facines, biologs

and other appropriate measures. Use of impervious surfaces for stabilization should be

avoided.

(a) A record of the dates when major grading activities occur, when construction

activities temporarily or permanently cease on a portion of the site, and when

stabilization measures are initiated shall be maintained and included in the SWPPP.

(b) Except as provided in Section II D 2 a (1) (c), (d) and (e), stabilization measures shall

be initiated as soon as practicable in portions of the site where construction activities

have temporarily or permanently ceased, but in no case more than seven days after the

construction activity in that portion of the site has temporarily or permanently ceased.

(c) Where the initiation of stabilization measures by the seventh day after construction

activity temporary or permanently ceased is precluded by snow cover or frozen ground

conditions, stabilization measures shall be initiated as soon as practicable.

(d) Where construction activity on a portion of the site is temporarily ceased, and earth

disturbing activities will be resumed within 30 days, temporary stabilization measures do

not have to be initiated on that portion of the site.

(e) In drought-stricken areas where initiating perennial vegetative stabilization measures

is not possible within seven days after construction activity has temporarily or

permanently ceased, final vegetative stabilization measures shall be initiated as soon as

practicable.

(2) Structural practices. The SWPPP shall include a description of structural practices to

divert flows from exposed soils, retain/detain flows or otherwise limit runoff and the

discharge of pollutants from exposed areas of the site. Such practices may include, but

are not limited to: silt fences, earth dikes, drainage swales, sediment traps, check dams,

subsurface drains, pipe slope drains, level spreaders, storm drain inlet protection, rock

outlet protection, reinforced soil retaining systems, gabions, and temporary or permanent

sediment basins. Structural practices should be located on upland soils to the degree

attainable. The department and the permit issuing authority encourages the use of a

combination of erosion and sediment control measures in order to achieve maximum

pollutant removal.

(a) Sediment basins: For common drainage locations that serve an area with three or more

acres disturbed at one time, a temporary (or permanent) sediment basin providing 3,618

cubic feet of storage per acre drained, or equivalent control measures, shall be provided

where attainable until final stabilization of the site. The 3,618 cubic feet of storage area

per acre drained does not apply to flows from off-site areas and flows from on-site areas

that are either undisturbed or have undergone final stabilization where such flows are

diverted around both the disturbed area and the sediment basin. In determining whether

installing a sediment basin is attainable, the permittee may consider factors such as site

soils, slope, available area on site, etc. In any event, the permittee must consider public

safety, especially as it relates to children, as a design factor for the sediment basin and

alternative sediment controls shall be used where site limitations would preclude a safe

design.

(b) For drainage locations which serve three or more acres at one time and where a

temporary sediment basin or equivalent controls is not attainable, smaller sediment basins

and/or sediment traps should be used. At a minimum, silt fences, vegetative buffer strips,

or equivalent sediment controls are required for all down slope boundaries, and for those

side slope boundaries deemed appropriate as dictated by individual site conditions.

(c) For drainage locations serving less than three acres, smaller sediment basins or

sediment traps or both should be used. At a minimum, silt fences, vegetative buffer strips

or equivalent sediment controls are required for all downslope boundaries, and for those

side slope boundaries deemed appropriate as dictated by individual site conditions, of the

construction area unless a sediment basin providing storage for 3,618 cubic feet of

storage per acre drained is provided.

b. Management practices.

(1) All control measures must be properly selected, installed, and maintained in

accordance with manufacturer specifications and good engineering practices. If periodic

inspections or other information indicates a control has been used inappropriately, or

incorrectly, the permittee must replace or modify the control for site situations as soon as

practicable.

(2) If sediment escapes the construction site, off-site accumulations of sediment must be

removed at a frequency sufficient to minimize off-site impacts.

(3) Litter, construction debris, and construction chemicals exposed to stormwater shall be

prevented from becoming a pollutant source in stormwater discharges.

c. Stormwater management.

(1) The SWPPP shall include a description of, and all necessary calculations supporting,

all post-construction stormwater management measures that will be installed during the

construction process to control pollutants in stormwater discharges after construction

operations have been completed. Structural measures should be placed on upland soils to

the degree attainable. Such measures must be designed and installed in accordance with

applicable local and/or state requirements.

(2) Such measures may include, but are not limited to: stormwater detention structures

(including dry ponds); stormwater retention structures; flow attenuation by use of open

vegetated swales and natural depressions; infiltration of runoff on-site; stormwater

wetlands; sand filters; bioretention systems; water quality structures; and sequential

systems (which combine several practices). The SWPPP shall include an explanation of

the technical basis used to select the practices to control pollution and flows that exceed

predevelopment levels.

(3) Outflows from a stormwater management facility or stormwater conveyance system

shall be discharged to an adequate channel. In addition, the natural, physical, chemical,

and biological characteristics and functions of the receiving waters must be maintained

and protected (e.g., no significant changes in the hydrological regime of the receiving

water).

d. Other controls.

(1) The SWPPP shall describe measures to prevent the discharge of solid materials,

including building materials, garbage, and debris to surface waters of the state, except as

authorized by a Clean Water Act § 404 permit.

(2) Where construction vehicle access routes intersect paved public roads, provisions

shall be made to minimize the transport of sediment by vehicular tracking onto the paved

surface. Where sediment is transported onto a public road surface, the road shall be

cleaned thoroughly at the end of each day. Sediment shall be removed from the roads by

shoveling or sweeping and transported to a sediment control disposal area. Street

washing shall be allowed only after sediment is removed in this manner.

(3) The SWPPP shall ensure and demonstrate compliance with applicable state or local

waste disposal, sanitary sewer or septic system regulations.

(4) The SWPPP shall include a description of construction and waste materials expected

to be stored on-site with updates as appropriate. The plan shall also include a description

of controls to reduce pollutants from these materials, including storage practices to

minimize exposure of the materials to stormwater, and for spill prevention and response.

(5) The SWPPP shall include a description of pollutant sources from areas other than

construction (including stormwater discharges from dedicated asphalt plants and

dedicated concrete plants), and a description of controls and measures that will be

implemented at those sites to minimize pollutant discharges.

e. Applicable state or local programs.

The SWPPP shall be consistent with all applicable state or local requirements for erosion

and sediment control and stormwater management including updates to the SWPPP as

necessary to reflect any revisions to applicable state or local requirements for erosion and

sediment control and stormwater management.

3. Maintenance of controls.

a. The SWPPP must include a description and schedule of procedures to maintain in good

and effective operating conditions vegetation, erosion and sediment control measures and

other protective measures during construction identified in the site plan. If site

inspections required by Section II D 4 identify BMPs that are not operating effectively,

maintenance shall be performed before the next anticipated storm event, or as soon as

practicable to maintain the continued effectiveness of stormwater controls.

b. If existing BMPs need to be modified or if additional BMPs are necessary for any

reason, implementation shall be completed before the next anticipated storm event. If

implementation before the next anticipated storm event is impracticable, the situation

shall be documented in the SWPPP and alternative BMPs shall be implemented as soon

as practicable.

4. Inspections. Inspections by qualified personnel must be conducted of all areas of the

site disturbed by construction activity, and areas used for storage of materials that are

exposed to stormwater. "Qualified personnel" means a licensed professional engineer,

responsible land disturber (RLD), or other knowledgeable person who (i) holds a

certificate of competence from the board in the area of project inspection; or (ii) is

enrolled in the board’s training program for project inspection or combined administrator

and successfully completes such program within one year of enrollment.

a. Inspections shall be conducted at least once every 14 calendar days and within 48

hours of the end of any runoff producing storm event. Where areas have been finally or

temporarily stabilized or runoff is unlikely due to winter conditions (e.g., the site is

covered with snow or ice, or frozen ground exists) such inspections shall be conducted at

least once every month.

b. Inspectors must look for evidence of, or the potential for, pollutants entering the

stormwater conveyance system. Erosion and sediment control measures identified in the

SWPPP shall be observed to ensure proper operation. Discharge locations where

accessible shall be inspected to ascertain whether erosion and sediment control measures

are effective in preventing significant impacts to receiving waters. Where discharge

locations are inaccessible, nearby downstream locations shall be inspected to the extent

that such inspections are practicable. Locations where vehicles enter or exit the site shall

be inspected for evidence of off-site sediment tracking.

c. Utility line installation, pipeline construction, and other examples of long, narrow,

linear construction activities may limit the access of inspection personnel to the areas

described in Section II D 4 b. Inspection of these areas could require that vehicles

compromise temporarily or even permanently stabilized areas, cause additional

disturbance of soils, and increase the potential for erosion. In these circumstances,

controls must be inspected on the same frequencies as other construction projects, but

representative inspections may be performed. For representative inspections, personnel

must inspect controls along the construction site for 0.25 miles above and below each

access point where a roadway, undisturbed right-of-way, or other similar feature

intersects the construction site and allows access to the areas described above. The

conditions of the controls along each inspected 0.25-mile segment may be considered as

representative of the condition of controls along that reach extending from the end of the

0.25-mile segment to either the end of the next 0.25-mile segment, or to the end of the

project, whichever occurs first. Inspection locations must be listed in the report required

by Section II D 4 e.

d. Based on the results of the inspection, the site and activity description identified in the

plan in accordance with Section II D 1 of this permit and pollution prevention measures

identified in the SWPPP in accordance with Section II D 2 of this permit shall be revised

as appropriate within seven calendar days following the inspection.

e. A report summarizing the scope of the inspection, names and qualifications of

personnel making the inspection, the dates of the inspection, major observations relating

to the implementation of the SWPPP, and actions taken in accordance with Section II D 4

d of the permit shall be made and retained as part of the SWPPP in accordance with

Section III B of this permit. Major observations should include:

(1) The location(s) of discharges of sediment or other pollutants from the site;

(2) Location(s) of BMPs that need to be maintained;

(3) Location(s) of BMPs that failed to operate as designed or proved inadequate for a

particular location;

(4) Location(s) where additional BMPs are needed that did not exist at the time of

inspection; and

(5) Corrective action required including any changes to the SWPPP that are necessary

and implementation dates.

The reports shall identify any incidents of noncompliance. Where a report does not

identify any incidents of noncompliance, the report shall contain a certification that the

facility is in compliance with the stormwater pollution prevention plan and this permit.

The report shall be signed in accordance with Section III K of this permit.

5. Nonstormwater discharge management. The SWPPP shall identify all allowable

sources of nonstormwater discharges listed in Section I D 2 of this permit that are

combined with stormwater discharges from the construction activity at the site, except for

flows from fire fighting activities. The SWPPP shall identify and ensure the

implementation of appropriate pollution prevention measures for the nonstormwater

components of the discharge.

SECTION III

CONDITIONS APPLICABLE TO ALL VSMP PERMITS

NOTE: Monitoring is not required for this permit. If you choose to monitor your

stormwater discharges or BMPs, you must comply with the requirements of subsections

A, B, and C, as appropriate.

A. Monitoring.

1. Samples and measurements taken for the purpose of monitoring shall be representative

of the monitoring activity.

2. Monitoring shall be conducted according to procedures approved under 40 CFR Part

136 or alternative methods approved by the U.S. Environmental Protection Agency,

unless other procedures have been specified in this permit.

3. The permittee shall periodically calibrate and perform maintenance procedures on all

monitoring and analytical instrumentation at intervals that will ensure accuracy of

measurements.

B. Records.

1. Records of monitoring information shall include:

a. The date, exact place, and time of sampling or measurements;

b. The individual(s) who performed the sampling or measurements;

c. The date(s) and time(s) analyses were performed;

d. The individual(s) who performed the analyses;

e. The analytical techniques or methods used; and

f. The results of such analyses.

2. The permittee shall retain records of all monitoring information, including all

calibration and maintenance records and all original strip chart recordings for continuous

monitoring instrumentation, copies of all reports required by this permit, and records of

all data used to complete the registration statement for this permit, for a period of at least

three years from the date of the sample, measurement, report or request for coverage.

This period of retention shall be extended automatically during the course of any

unresolved litigation regarding the regulated activity or regarding control standards

applicable to the permittee, or as requested by the board.

C. Reporting monitoring results.

1. The permittee shall submit the results of the monitoring required by this permit not

later than the 10th day of the month after monitoring takes place, unless another reporting

schedule is specified elsewhere in this permit. Monitoring results shall be submitted to

the permit issuing authority.

2. Monitoring results shall be reported on a discharge monitoring report (DMR) or on

forms provided, approved or specified by the department.

3. If the permittee monitors any pollutant specifically addressed by this permit more

frequently than required by this permit using test procedures approved under 40 CFR Part

136 or using other test procedures approved by the U.S. Environmental Protection

Agency or using procedures specified in this permit, the results of this monitoring shall

be included in the calculation and reporting of the data submitted in the DMR or

reporting form specified by the department.

4. Calculations for all limitations which require averaging of measurements shall utilize

an arithmetic mean unless otherwise specified in this permit.

D. Duty to provide information. The permittee shall furnish to the permit issuing

authority, within a reasonable time, any information which the permit issuing authority

may request to determine whether cause exists for modifying, revoking and reissuing, or

terminating this permit or to determine compliance with this permit. The permit issuing

authority may require the permittee to furnish, upon request, such plans, specifications,

and other pertinent information as may be necessary to determine the effect of the wastes

from his discharge on the quality of state waters, or such other information as may be

necessary to accomplish the purposes of the Virginia Stormwater Management Act . The

permittee shall also furnish to the permit issuing authority, upon request, copies of

records required to be kept by this permit.

E. Compliance schedule reports. Reports of compliance or noncompliance with, or any

progress reports on, interim and final requirements contained in any compliance schedule

of this permit shall be submitted no later than 14 days following each schedule date.

F. Unauthorized discharges. Except in compliance with this permit or another permit

issued by the permit issuing authority or the Department of Environmental Quality, it

shall be unlawful for any person to:

1. Discharge into state waters sewage, industrial wastes, other wastes, or any noxious or

deleterious substances; or

2. Otherwise alter the physical, chemical or biological properties of such state waters and

make them detrimental to the public health, or to animal or aquatic life, or to the use of

such waters for domestic or industrial consumption, or for recreation, or for other uses.

G. Reports of unauthorized discharges. Any permittee who discharges or causes or allows

a discharge of sewage, industrial waste, other wastes or any noxious or deleterious

substance into or upon state waters in violation of Section III F, or who discharges or

causes or allows a discharge that may reasonably be expected to enter state waters in

violation of Section III F, shall notify the department, the Department of Environmental

Quality, and the permit issuing authority of the discharge immediately upon discovery of

the discharge, but in no case later than 24 hours after said discovery. A written report of

the unauthorized discharge shall be submitted to the department, the Department of

Environmental Quality, and the permit issuing authority within five days of discovery of

the discharge. The written report shall contain:

1. A description of the nature and location of the discharge;

2. The cause of the discharge;

3. The date on which the discharge occurred;

4. The length of time that the discharge continued;

5. The volume of the discharge;

6. If the discharge is continuing, how long it is expected to continue;

7. If the discharge is continuing, what the expected total volume of the discharge will be;

And

8. Any steps planned or taken to reduce, eliminate and prevent a recurrence of the present

discharge or any future discharges not authorized by this permit.

Discharges reportable to the department, the Department of Environmental Quality, and

the permit issuing authority under the immediate reporting requirements of other

regulations are exempted from this requirement.

H. Reports of unusual or extraordinary discharges. If any unusual or extraordinary

discharge including a bypass or upset should occur from a facility and the discharge

enters or could be expected to enter state waters, the permittee shall promptly notify, in

no case later than 24 hours, the department, the Department of Environmental Quality,

and the permit issuing authority by telephone after the discovery of the discharge. This

notification shall provide all available details of the incident, including any adverse

effects on aquatic life and the known number of fish killed. The permittee shall reduce

the report to writing and shall submit it to the department, the Department of

Environmental Quality, and the permit issuing authority within five days of discovery of

the discharge in accordance with Part Section III I 2. Unusual and extraordinary

discharges include but are not limited to any discharge resulting from:

1. Unusual spillage of materials resulting directly or indirectly from processing

operations;

2. Breakdown of processing or accessory equipment;

3. Failure or taking out of service some or all of the facilities; and

4. Flooding or other acts of nature.

I. Reports of noncompliance. The permittee shall report any noncompliance which may

adversely affect state waters or may endanger public health.

1. An oral report shall be provided within 24 hours from the time the permittee becomes

aware of the circumstances. The following shall be included as information which shall

be reported within 24 hours under this paragraph:

a. Any unanticipated bypass; and

b. Any upset which causes a discharge to surface waters.

2. A written report shall be submitted within five days and shall contain:

a. A description of the noncompliance and its cause;

b. The period of noncompliance, including exact dates and times, and if the

noncompliance has not been corrected, the anticipated time it is expected to continue; and

c. Steps taken or planned to reduce, eliminate, and prevent reoccurrence of the

noncompliance.

The permit issuing authority may waive the written report on a case-by-case basis for

reports of noncompliance under Section III I if the oral report has been received within

24 hours and no adverse impact on state waters has been reported.

3. The permittee shall report all instances of noncompliance not reported under Section

III I 1 or 2 in writing at the time the next monitoring reports are submitted. The reports

shall contain the information listed in Section III I 2.

NOTE: The immediate (within 24 hours) reports required in Section III G, H and I may

be made to the department's Urban Program’s Section of the Division of Soil and Water

Conservation. Reports may be made by telephone or by fax. For reports outside normal

working hours, leaving a recorded message shall fulfill the immediate reporting

requirement. For emergencies, the Virginia Department of Emergency Management

maintains a 24 hour telephone service at 1-800-468-8892.

4. Where the permittee becomes aware that it failed to submit any relevant facts in a

permit application, or submitted incorrect information in a permit application or in any

report to the department, it shall promptly submit such facts or information.

J. Notice of planned changes.

1. The permittee shall give notice to the permit issuing authority as soon as possible of

any planned physical alterations or additions to the permitted facility. Notice is required

only when:

a. The permittee plans an alteration or addition to any building, structure, facility, or

installation from which there is or may be a discharge of pollutants, the construction of

which commenced:

(1) After promulgation of standards of performance under § 306 of the federal Clean

Water Act which are applicable to such source; or

(2) After proposal of standards of performance in accordance with § 306 of the Clean

Water Act which are applicable to such source, but only if the standards are promulgated

in accordance with § 306 within 120 days of their proposal;

b. The alteration or addition could significantly change the nature or increase the quantity

of pollutants discharged. This notification applies to pollutants which are not subject to

effluent limitations in this permit; or

2. The permittee shall give advance notice to the permit issuing authority of any planned

changes in the permitted facility or activity which may result in noncompliance with

permit requirements.

K. Signatory requirements.

1. Registration statement. All registration statements shall be signed as follows:

a. For a corporation: by a responsible corporate officer. For the purpose of this section, a

responsible corporate officer means: (i) a president, secretary, treasurer, or vice-president

of the corporation in charge of a principal business function, or any other person who

performs similar policy-making or decision-making functions for the corporation; or (ii)

the manager of one or more manufacturing, production, or operating facilities, provided

the manager is authorized to make management decisions that govern the operation of the

regulated facility including having the explicit or implicit duty of making major capital

investment recommendations, and initiating and directing other comprehensive measures

to assure long term environmental compliance with environmental laws and regulations;

the manager can ensure that the necessary systems are established or actions taken to

gather complete and accurate information for permit application requirements; and where

authority to sign documents has been assigned or delegated to the manager in accordance

with corporate procedures;

b. For a partnership or sole proprietorship: by a general partner or the proprietor,

respectively; or

c. For a municipality, state, federal, or other public agency: by either a principal

executive officer or ranking elected official. For purposes of this section, a principal

executive officer of a public agency includes: (i) the chief executive officer of the agency

or (ii) a senior executive officer having responsibility for the overall operations of a

principal geographic unit of the agency.

2. Reports, etc. All reports required by permits and other information requested by the

board shall be signed by a person described in Section III K 1 or by a duly authorized

representative of that person. A person is a duly authorized representative only if:

a. The authorization is made in writing by a person described in Section III K 1;

b. The authorization specifies either an individual or a position having responsibility for

the overall operation of the regulated facility or activity such as the position of plant

manager, operator of a well or a well field, superintendent, position of equivalent

responsibility, or an individual or position having overall responsibility for environmental

matters for the company. (A duly authorized representative may thus be either a named

individual or any individual occupying a named position); and

c. The written authorization is submitted to the department.

3. Changes to authorization. If an authorization under Section III K 2 is no longer

accurate because a different individual or position has responsibility for the overall

operation of the facility, a new authorization satisfying the requirements of Section III K

2 shall be submitted to the permit issuing authority prior to or together with any reports or

information to be signed by an authorized representative.

4. Certification. Any person signing a document under Section III K 1 or 2 shall make the

following certification:

"I certify under penalty of law that this document and all attachments were prepared

under my direction or supervision in accordance with a system designed to assure that

qualified personnel properly gather and evaluate the information submitted. Based on my

inquiry of the person or persons who manage the system, or those persons directly

responsible for gathering the information, the information submitted is, to the best of my

knowledge and belief, true, accurate, and complete. I am aware that there are significant

penalties for submitting false information, including the possibility of fine and

imprisonment for knowing violations."

L. Duty to comply. The permittee shall comply with all conditions of this permit. Any

permit noncompliance constitutes a violation of the Virginia Stormwater Management

Act and the Clean Water Act, except that noncompliance with certain provisions of this

permit may constitute a violation of the Virginia Stormwater Management Act but not the

Clean Water Act. Permit noncompliance is grounds for enforcement action; for permit

termination, revocation and reissuance, or modification; or denial of a permit renewal

application.

The permittee shall comply with effluent standards or prohibitions established under §

307(a) of the Clean Water Act for toxic pollutants within the time provided in the

regulations that establish these standards or prohibitions or standards for sewage sludge

use or disposal, even if this permit has not yet been modified to incorporate the

requirement.

M. Duty to reapply. If the permittee wishes to continue an activity regulated by this

permit after the expiration date of this permit, the permittee shall submit a new

registration statement at least 90 days before the expiration date of the existing permit,

unless permission for a later date has been granted by the board. The board shall not

grant permission for registration statements to be submitted later than the expiration date

of the existing permit.

N. Effect of a permit. This permit does not convey any property rights in either real or

personal property or any exclusive privileges, nor does it authorize any injury to private

property or invasion of personal rights, or any infringement of federal, state or local law

or regulations.

O. State law. Nothing in this permit shall be construed to preclude the institution of any

legal action under, or relieve the permittee from any responsibilities, liabilities, or

penalties established pursuant to any other state law or regulation or under authority

preserved by § 510 of the Clean Water Act. Except as provided in permit conditions on

"bypassing" (Section III U), and "upset" (Section III V) nothing in this permit shall be

construed to relieve the permittee from civil and criminal penalties for noncompliance.

P. Oil and hazardous substance liability. Nothing in this permit shall be construed to

preclude the institution of any legal action or relieve the permittee from any

responsibilities, liabilities, or penalties to which the permittee is or may be subject under

§§ 62.1-44.34:14 through 62.1-44.34:23 of the State Water Control Law.

Q. Proper operation and maintenance. The permittee shall at all times properly operate

and maintain all facilities and systems of treatment and control (and related

appurtenances) which are installed or used by the permittee to achieve compliance with

the conditions of this permit. Proper operation and maintenance also includes effective

plant performance, adequate funding, adequate staffing, and adequate laboratory and

process controls, including appropriate quality assurance procedures. This provision

requires the operation of back-up or auxiliary facilities or similar systems which are

installed by the permittee only when the operation is necessary to achieve compliance

with the conditions of this permit.

R. Disposal of solids or sludges. Solids, sludges or other pollutants removed in the

course of treatment or management of pollutants shall be disposed of in a manner so as to

prevent any pollutant from such materials from entering state waters.

S. Duty to mitigate. The permittee shall take all reasonable steps to minimize or prevent

any discharge in violation of this permit which has a reasonable likelihood of adversely

affecting human health or the environment.

T. Need to halt or reduce activity not a defense. It shall not be a defense for a permittee

in an enforcement action that it would have been necessary to halt or reduce the permitted

activity in order to maintain compliance with the conditions of this permit.

U. Bypass.

1. "Bypass" means the intentional diversion of waste streams from any portion of a

treatment facility. The permittee may allow any bypass to occur which does not cause

effluent limitations to be exceeded, but only if it also is for essential maintenance to

ensure efficient operation. These bypasses are not subject to the provisions of Section III

U 2 and 3.

2. Notice.

a. Anticipated bypass. If the permittee knows in advance of the need for a bypass, prior

notice shall be submitted, if possible at least 10 days before the date of the bypass.

b. Unanticipated bypass. The permittee shall submit notice of an unanticipated bypass as

required in Section III I.

3. Prohibition of bypass.

a. Bypass is prohibited, and the permit issuing authority may take enforcement action

against a permittee for bypass unless:

(1) Bypass was unavoidable to prevent loss of life, personal injury, or severe property

damage;

(2) There were no feasible alternatives to the bypass, such as the use of auxiliary

treatment facilities, retention of untreated wastes, or maintenance during normal periods

of equipment downtime. This condition is not satisfied if adequate back-up equipment

should have been installed in the exercise of reasonable engineering judgment to prevent

a bypass which occurred during normal periods of equipment downtime or preventive

maintenance; and

(3) The permittee submitted notices as required under Section III U 2.

b. The permit issuing authority may approve an anticipated bypass, after considering its

adverse effects, if the permit issuing authority determines that it will meet the three

conditions listed in Section III U 3 a.

V. Upset.

1. An upset constitutes an affirmative defense to an action brought for noncompliance

with technology-based permit effluent limitations if the requirements of Section III V 2

are met. A determination made during administrative review of claims that

noncompliance was caused by upset, and before an action for noncompliance, is not a

final administrative action subject to judicial review.

2. A permittee who wishes to establish the affirmative defense of upset shall demonstrate,

through properly signed, contemporaneous operating logs or other relevant evidence that:

a. An upset occurred and that the permittee can identify the cause(s) of the upset;

b. The permitted facility was at the time being properly operated;

c. The permittee submitted notice of the upset as required in Section III I; and

d. The permittee complied with any remedial measures required under Section III S.

3. In any enforcement proceeding, the permittee seeking to establish the occurrence of an

upset has the burden of proof.

W. Inspection and entry. The permittee shall allow the director as the Board’s designee,

or an authorized representative (including an authorized contractor acting as a

representative of the administrator), upon presentation of credentials and other documents

as may be required by law to:

1. Enter upon the permittee's premises where a regulated facility or activity is located or

conducted, or where records must be kept under the conditions of this permit;

2. Have access to and copy, at reasonable times, any records that must be kept under the

conditions of this permit;

3. Inspect at reasonable times any facilities, equipment (including monitoring and control

equipment), practices, or operations regulated or required under this permit; and

4. Sample or monitor at reasonable times, for the purposes of ensuring permit compliance

or as otherwise authorized by the Clean Water Act and the Virginia Stormwater

Management Act, any substances or parameters at any location.

For purposes of this section, the time for inspection shall be deemed reasonable during

regular business hours, and whenever the facility is discharging. Nothing contained

herein shall make an inspection unreasonable during an emergency.

X. Permit actions. Permits may be modified, revoked and reissued, or terminated for

cause. The filing of a request by the permittee for a permit modification, revocation and

reissuance, or termination, or a notification of planned changes or anticipated

noncompliance does not stay any permit condition.

Y. Transfer of permits.

1. Permits are not transferable to any person except after notice to the permit issuing

authority. Except as provided in Section III Y 2, a permit may be transferred by the

permittee to a new owner or operator only if the permit has been modified or revoked and

reissued, or a minor modification made, to identify the new permittee and incorporate

such other requirements as may be necessary under the Virginia Stormwater Management

Act and the Clean Water Act.

2. As an alternative to transfers under Section III Y 1, this permit may be automatically

transferred to a new permittee if:

a. The current permittee notifies the permit issuing authority at least 30 days in advance

of the proposed transfer of the title to the facility or property;

b. The notice includes a written agreement between the existing and new permittees

containing a specific date for transfer of permit responsibility, coverage, and liability

between them; and

c. The permit issuing authority does not notify the existing permittee and the proposed

new permittee of its intent to modify or revoke and reissue the permit. If this notice is

not received, the transfer is effective on the date specified in the agreement mentioned in

Section III Y 2 b.

Z. Severability. The provisions of this permit are severable, and if any provision of this

permit or the application of any provision of this permit to any circumstance, is held

invalid, the application of such provision to other circumstances and the remainder of this

permit shall not be affected thereby.