VIRGINIA DEPARTMENT OF TRANSPORTATION

LOCATION AND DESIGN DIVISION

INSTRUCTIONAL AND INFORMATIONAL MEMORANDUM

| GENERAL SUBJECT: #### ROADSIDE DEVELOPMENT | NUMBER: #### IIM-LD-122.11 |
| --- | --- |
| SPECIFIC SUBJECT: ROADSIDE DEVELOPMENT SHEET; COORDINATION; COMPUTING QUANTITIES/SUMMARIZATION | #### DATE: JANUARY 13, 2005 |
|  | SUPERSEDES: #### IIM-LD-122.10 |
| DIVISION ADMINISTRATOR APROVAL: Mohammad Mirshahi, PE Approved January 13, 2005 |  |

Changes are shaded.

CURRENT REVISION

- Updated primary points of contact.

- Added Pay Item for Erosion Control Mulch in S.Y./m2.

EFFECTIVE DATE

- These instructions are effective upon receipt.

POLICY

- The Roadside Development Sheet and the Erosion Control Summary Sheet are to be included in project plan assemblies.

- The Roadside Development Summary will indicate the Asset Management Division’s recommended seed mixtures, and estimated quantities for Topsoil, Seeding (Regular and Legume), Fertilizer and Lime.

- Seed additives (e.g. foxtail millet) are paid for as Regular Seeding except Crown Vetch, Sericea Lespedeza and Birdsfoot Trefoil

- Seed mixture recommendations may at times deviate from the seed mixture guidelines on the Roadside Development Sheet.  The District Roadside Manager will provide recommendations for the application of seed mixtures (core mix and additives), fertilizer, lime, etc.

SPECIAL INSTRUCTIONS

- The approximate area (hectares or acres) to be disturbed will be shown under “Notes” on the Roadside Development Sheet.  This area is not to be expanded for estimating purposes.

- Notes on the Roadside Development Sheet marked by a star are for the use of field forces only.  The Designer is not to use any percentages shown under “Notes” on the Roadside Development sheet when computing quantities.

MULCH

- Roadside Development involves two categories of mulch as follows:

- Seeding Mulch, Type I or II is applied in the field with the seed mixture.  This mulch is included in the price for the regular seeding and is not summarized in the plans.

- Erosion Control Mulch is summarized on the Erosion Control Summary Sheet when recommended by the Asset Management Division.  This material is estimated at the rate of 0.25 acres (1,210 S.Y.) per 100 feet of alignment or 0.332 hectares (3,319 m2) per 100 meters of roadway alignment) and is to be paid for as follows:

PAY ITEM			UNIT			ITEM CODE

Erosion Control Mulch	Acres (Hectare)			27288

Erosion Control Mulch	  S.Y.     (m2)				27284

LEGUME SEEDING

- The seed mixes available for roadside development include three “Legume” seeds, Crown Vetch, Sericea Lespedeza and Birdsfoot Trefoil specified as additives “E, F. and G” on the Roadside Development Sheet.

- These Legume seeds are used only on slopes 3:1 or greater and are not used on shoulders or other locations to be mowed.

- Legume Seed, and Legume Overseeding are to be summarized for separate payment.

- Whenever the Asset Management Division specifies any of these Legume seeds, the mowable areas on the project (slopes flatter than 3:1) and non-mowable areas (slopes 3:1 and greater) must be measured separately in order to accurately summarize the seeding requirements.

ESTIMATING QUANTITIES

- If the lime application rate is not provided by the Asset Management Division, the Designer should estimate the Normal Lime Quantity based on 5 metric tons per hectare (2 tons per acre).

- If the fertilizer application rate is not provided by the Asset Management Division, the Designer should estimate the Normal Fertilizer Quantity based on 675 kilograms per hectare (600 pounds per acre).

- The seed mixtures (core mix plus additives) shown on the Roadside Development Sheet are weights per hectare (or acre) of disturbed area.  These quantities may vary for each construction season.

- The Designer is advised to:

- Determine the disturbed area to be seeded.

- Determine the application rate for the sloped and mowed areas shown for each construction season.

Example for Seed Mix 2E:

100 lbs. Core Mix + 20 lbs. Additive =120 lbs.

- The greatest seeding rate is assumed to be the “Normal” Seeding rate.

Example for 10 acre area:

MIX REQUIREMENTS ON THIS PROJECT

| PROJECT NUMBERS | SLOPES |  | MOWED | SLOPES | MOWED | SLOPES | MOWED |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPRING & FALL |  |  | SUMMER |  | LATE FALL & WINTER |  |
| 0123-123-103 | 2E | 2B |  | 3A | 3A | 4B | 4B |

120 LBS.	120 LBS.	110 LBS     110 LBS.      120 LBS.      120 LBS.

The Normal Seeding rate = 120 lbs. per acre.

120 lbs. x 10 acres of disturbed area = 1200 lbs. “Normal” Seeding Quantity

- When a legume seed additive is specified (Crown Vetch, Sericea Lespedeza or Birdsfoot Trefoil) the sloped areas and mowed areas must be measured separately when summarizing seeding quantities.

- Determine the flat (less than 3:1) areas and sloped (3:1 and greater) areas to be seeded.

Example: 10 acres of mowed areas; 5 acres of sloped areas.

- Determine the application rate for the mowed areas.

Example for “Seed Mix 2B”:100 lbs. Core Mix + 20 lbs. Additive = 120 lbs.

- Determine the application rate for the sloped areas:

Example for Seed Mix 2E: Core Mix “2” = 100 lbs.; Additive E” = 20 lbs.

- Determine the quantities of Regular Seed and Legume Seed.

Example for mowed area (Seed Mix 2B):

Core Mix 100 lbs. + 20 lbs. = 120 lbs. x 10 acres = 1200 lbs. Regular Seed

Example for sloped areas (Seed Mix 2E):

100 lbs. x 5 acres = 500 lbs. Regular Seed

20 lbs. x 5 acres = 100 lbs. Legume Seed

- The “Normal” quantities for lime, fertilizer, and seeding are based on the actual area to be disturbed.  The “Normal” quantities are to be increased by the following percentage factors to obtain the quantity to show in the summary:

- Lime = Normal Quantity increased by 90%

- Fertilizer (15-30-15)= Normal Quantity increased by 90%

- Regular Seed = Normal Seeding Quantity increased by 60%

- Overseeding=100% of Normal Seeding Quantity (no mulch or fertilizer)

- Legume Seed = Normal Seeding Quantity increased by 60%

- Legume Overseeding = 100% of Normal Seeding Quantity (no mulch or fertilizer)

Examples for determining quantities to summarize:

20 tons “normal” Lime x 1.90 (or 190%) = 38 tons Lime

3 tons “normal” Fertilizer x 1.90 (or 190%) = 5.7 or 6 tons Fertilizer

1700 lbs.“normal”Seeding x 1.60 (or 160%) = 2720 lbs. Regular Seeding

1700 lbs. “normal” Seeding (@ 100%) =  1700 lbs. Overseeding

100 lbs. “normal” Legume Seeding x 1.60 (or 160%)= 160 lbs. Legume Seed

100 lbs. “normal” Legume Seeding (@ 100%) = 100 lbs. Legume Overseeding

PAY ITEMS

- Lime			Metric Ton/Ton			27250

- Fertilizer			Metric Ton/Ton			27215

- Regular Seed		kg/lbs.					27102

- Overseeding		kg/lbs.					27103

- Legume Seed		kg/lbs.					27104

- Legume Overseeding	kg/lbs.					27105

- Topsoil Class A		ha/acres				27012

- Topsoil Class B		ha/acres				27022

- Erosion Control Mulch	ha/acres/m2/S.Y.			27288

REVIEW BY ASSET MANAGEMENT DIVISION

- The Roadside Development Sheet is to be reviewed by the Asset Management Division prior to submission of the plan assembly for construction.

- Anytime the current Roadside Development Sheet is replaced by a revised Roadside Development Sheet, the District Roadside Manager should be requested to determine the need for any changes in seed mixes, quantities, etc.

INSERTABLE SHEETS

- The Roadside Development Sheet may be obtained through the CADD Insertable Sheet Directory.

- Special Design Section Drawing No. A-4 (Imperial)

- Special Design Section Drawing No. MA-4 (Metric)

- The Erosion Control Summary Sheet may be obtained through the CADD Insertable Sheet Directory.

- Special Design Section Drawing No. A-5 (Imperial)

- Special Design Section Drawing No. MA-5 (Metric)