## Appendix 6H-3	Runoff Curve Numbers for Urban Areas1

Cover description							Curve numbers for hydrologic soil groups

Cover type and 							Average percent		A	B	C	D

hydrologic condition						impervious area2

Fully developed urban areas (vegetation established)

Open space (lawns, parks, golf courses, cemeteries, etc.)3

Poor condition (grass cover <50%)					68	79	86	89

Fair condition (grass cover 50% to 75%)				49	69	79	84

Good condition (grass cover > 75%)					39	61	74	80

Impervious areas:

Paved parking lots, roofs, driveways, etc.

(excluding right-of-way)									98	98	98	98

Streets and roads:

Paved; curbs and storm drains (excluding

right-of-way)											98	98	98	98

Paved; open ditches (including right-of-way)				83	89	92	93

Gravel (including right-of-way)						76	85	89	91

Dirt (including right-of-way)							72	82	87	89

Western desert urban areas:

Natural desert landscaping (pervious areas only)			63	77	85	88

Artificial desert landscaping (impervious weed barrier,

desert shrub with 25- to 50-mm sand or gravel mulch and

basin borders)										96	96	96	96

Urban districts:

Commercial and business 					85		89	92	94	95

Industrial										72		81	88	91	93

Residential districts by average lot size:

0.10 or less (town houses)					65		77	85	90	92

1/4 acre										38		61	75	83	87

1/3 acre										30		57	72	81	86

1/2 acre										25		54	70	80	85

1 acre											20		51	68	79	84

2 acre											12		46	65	77	82

Developing urban areas

Newly graded areas (pervious areas

only, no vegetation)										77	86	91	94

Idle lands (CNs are determined using cover types similar to those in Table 6 - 11).

Comments:

1 Average runoff condition, and Ia = 0.2S

2 The average percent impervious area shown was used to develop the composite CNs.  Other assumptions are as follows: impervious areas are directly connected to the drainage system, impervious areas have a CN of 98, and pervious areas are considered equivalent to open space in good hydrologic condition.  If the impervious area is not connected, the SCS method has an adjustment to reduce the effect.

3 CNs shown are equivalent to those of pasture.  Composite CNs may be computed for other combinations of open space cover type.

Source:	SCS-TP-149