## Appendix 6H-4	Runoff Curve Numbers for

## Cultivated Agricultural Areas

Cover description					Curve numbers for hydrologic soil group

Cover	Treatment2		Hydrologic		A	B	C	D

type                             		condition3

Fallow	Bare soil			-		 		77	86	91	94

Crop residue		Poor			76	85	90	93

cover (CR)		Good			74	83	88	90

Row  	Straight row (SR)	Poor			72	81	88	91

Crops							Good			67	78	85	89

SR + CR			Poor			71	80	87	90

Good			64	75	82	85

Contoured (C)		Poor			70	79	84	88

Good			65	75	82	86

C + CR 			Poor			69	78	83	87

Good			64	74	81	85

Contoured & 		Poor			66	74	80	82

terraced (C & T)		Good			62	71	78	81

C&T + CR		Poor			65	73	79	81

Good			61	70	77	80

Small grain   SR	Poor			65	76	84	88

Good			63	75	83	87

SR + CR			Poor			64	75	83	86

Good			60	72	80	84

C					Poor			63	74	82	85

Good			61	73	81	84

C + CR			Poor			62	73	81	84

Good			60	72	80	83

C&T				Poor			61	72	79	82

Good			59	70	78	81

C&T + CR		Poor			60	71	78	81

Good			58	69	77	80

Close-seeded  SR	Poor			66	77	85	89

or broadcast		Good			58	72	81	85

Legumes or C		Poor			64	75	83	85

Rotation			Good			55	69	78	83

Meadow  C&T		Poor			63	73	80	83

Good			51	67	76	80

Comments:

1 Average runoff condition, and Ia = 0.2S.

2 Crop residue cover applies only if residue is on at least 5% of the surface throughout the year.

3 Hydrologic condition is based on a combination of factors that affect infiltration and runoff, including (a) density and canopy of vegetative areas, (b) amount of year-round cover, (c) amount of grass or closed-seeded legumes in rotations, (d) percent of residue cover on the land surface (good > 20%) and (e) degree of roughness.

Poor:  Factors impair infiltration and tend to increase runoff.

Good: Factors encourage average and better than average infiltration and tend to decrease runoff.

Source:	SCS-TP-149