## Appendix 6H-5	Runoff Curve Numbers for

## Other Agricultural Areas

Cover description								Curve numbers for hydrologic soil group

Cover type				  	Hydrologic		A	B	C	D

condition

Pasture, grassland, or			Poor			68	79	86	89

range-continuous forage		Fair				49	69	79	84

for grazing2					Good			39	61	74	80

Meadow—continuous grass, —					30	58	71	78

protected from grazing and

generally mowed for hay

Brush—brush-weed-grass		Poor			48	67	77	83

mixture with brush the 			Fair				35	56	70	77

major element3				Good			430	48	65	73

Woods—grass combination		Poor			57	73	82	86

(orchard or tree farm)5			Fair				43	65	76	82

Good			32	58	72	79

Woods6								Poor			45	66	77	83

Fair				36	60	73	79

Good	    		430	55	70	77

Farmsteads—buildings, 			—				59	74	82	86

lanes, driveways and

surrounding lots

Comments:

1 Average runoff condition, and Ia = 0.2S

2 Poor: < 50% ground cover or heavily grazed with no mulch

Fair: 50 to 75% ground cover and not heavily grazed

Good: > 75% ground cover and lightly or only occasionally grazed

3 Poor: < 50% ground cover

Fair: 50 to 75% ground cover

Good: > 75% ground cover

4 Actual curve number is less than 30; use CN = 30 for runoff computations.

5 CNs shown were computed for areas with 50% grass (pasture) cover.  Other combinations of conditions may be computed from CNs for woods and pasture.

6 Poor: Forest litter, small trees and brush are destroyed by heavy grazing or regular burning.

Fair: Woods grazed but not burned, and some forest litter covers the soil.

Good: Woods protected from grazing, litter and brush adequately cover soil.

#### Source:	SCS-TP-149