## Appendix 6H-6	Conversion from Average Antecedent

## Moisture Condition to

## Dry and Wet Conditions

CN for Average Conditions	Corresponding CNs for

Dry 	Wet

100	100	100

95	87	98

90	78	96

85	70	94

80	63	91

75	57	88

70	51	85

65	45	82

60	40	78

55	35	74

50	31	70

45	26	65

40	22	60

35	18	55

30	15	50

25	12	43

15	6	30

5	2	13

Source:	USDA Soil Conservation Service TP-149 (SCS-TP-149), "A Method for Estimating Volume and Rate of Runoff in Small Watersheds," revised April 1973.