## Appendix 7D-1	Values of Roughness

## Coefficient n (Uniform Flow)

Type of Channel and Description	     Minimum	       Normal	     Maximum

_____________________________________________________________________________

LINED CHANNELS (Selected linings)

a.  Concrete					   		1. Trowel finish 	    0.011 	   0.013	    0.015

2.  Float finish	    0.013	   0.015	0.016		3.  Gunite, good section	    0.016	   0.019	    0.023

b.  Asphalt

1. Smooth					          0.013	          0.013		-

2. Rough	    0.016	    0.016		-

EXCAVATED OR DREDGED

a. Earth, straight and uniform

1.	Clean, recently completed	0.016	0.018	0.020

2.	Clean, after weathering	0.018	0.022	0.025

3.	Gravel, uniform section, clean	0.022	0.025	0.030

4.	With short grass, few weeds	0.022	0.027	0.033

b. Earth, winding and sluggish

1.	No vegetation	0.023	0.025	0.030

2.	Grass, some weeds	0.025	0.030	0.033

3.	Dense weeds or aquatic plants in deep channels	0.030	0.035	0.040

4.	Earth bottom and rubble sides	0.025	0.030	0.035

5.	Stony bottom and weedy sides	0.025	0.035	0.045

6.	Cobble bottom and clean sides	0.030	0.040	0.050

c. Dragline excavated or dredged

1.	No vegetation	0.025	0.028	0.033

2.	Light brush on banks	0.035	0.050	0.060

d. Rock cuts

1.	Smooth and uniform	0.025	0.035	0.040

2.	Jagged and irregular	0.035	0.040	0.050

e. Channels not maintained, weeds and brush uncut

1.	Dense weeds, high as flow depth	0.050	0.080	0.120

2.	Clean bottom, brush on sides	0.040	0.050	0.080

3.	Same, highest stage of flow	0.045	0.070	0.110

4.	Dense brush, high stage	0.080	0.100	0.140

NATURAL STREAMS

1. Minor streams (top width at flood stage <100 ft)

a.	Streams on Plain

1.	Clean, straight, full stage,

no rifts or deep pools	0.025	0.030	0.033

2.	Same as above, but more stones/weeds	0.030	0.035	0.040

3.	Clean, winding, some pools/shoals	0.033	0.040	0.045

4.	Same as above, but some weeds/stones	0.035	0.045	0.050

5.	Same as above, lower stages,

more ineffective slopes and sections	0.040	0.048	0.055

6.	Same as 4, but more stones	0.045	0.050	0.060

7.	Sluggish reaches, weedy, deep pools	0.050	0.070	0.080

8.	Very weedy reaches, deep pools, or								floodways with heavy stand of timber

and underbrush	0.075	0.100	0.150

b. Mountain streams, no vegetation in channel,

banks usually steep, trees and brush along

banks submerged at high stages

1.	Bottom: gravels, cobbles and few	0.030	0.040	0.050

boulders

2.	Bottom: cobbles with large boulders	0.040	0.050	0.070

## Appendix 7D-1	Values of Roughness

## Coefficient n (Uniform Flow)

Type of Channel and Description	  Minimum	       Normal	     Maximum

2. Floodplains

a.  Pasture, no brush

1. Short grass	0.025	0.030	0.035

2.	High grass	0.030	0.035	0.050

b.  Cultivated area

1.	No crop	0.020	0.030	0.040

2. Mature row crops	0.025	0.035	0.045

3.	Mature field crops	0.030	0.040	0.050

c.   Brush

1.	Scattered brush, heavy weeds	0.035	0.050	0.070

2.	Light brush and trees, in winter	0.035	0.050	0.060

3.	Light brush and trees, in summer	0.040	0.060	0.080

4.	Medium to dense brush, in winter	0.045	0.070	0.110				5.	Medium to dense brush, in summer	0.700	0.100	0.160

d.  Trees

1.	Dense Willows, summer, straight	0.110	0.150	0.200

2.Cleared land with tree stumps, no

sprouts	0.030	0.040	0.050

3. Same as above, but with heavy

growth of sprouts	0.050	0.060	0.080

4. Heavy stand of timber, a few down							trees, little undergrowth, flood stage

below branches	0.080	0.100	0.120

5.	Same as above, but with flood stage	0.100	0.120	0.160

reaching branches

3.   Major Streams (top width at flood stage > 100 ft)

The n-value is less than that for minor streams of

similar description, because banks offer less effective

resistance.

a.  Regular section with no boulders or brush	0.025	    -	0.060

b.  Irregular and rough section	0.035	    -	0.100

Source:	Chow, V.T.