## Appendix 7D-2	Recommended Maximum Water Velocities

## and Manning’s n as a Function of Soil Type

## and Flow Depth

| ASSHTO Classification | ASSHTO #### Soil Description | Fortier and Scobey Soil Description | Maximum Water Velocity (ft/s) | Manning's n -Flow Depth 0.5-2.0 ft |
| --- | --- | --- | --- | --- |
|  | BROKEN ROCK and COBBLES | Cobbles and Shingles | 5.5 | 0.030 |
| A-1-a | Stone fragments or GRAVEL, with or without well-graded 1 binder2 | Coarse gravel, non-colloidal | 4.5 | 0.025 |
| same | same | Fine gravel | 3.5 | 0.020 |
| A-1-b | Coarse SAND, with or without well-graded 1 binder2 | Graded loam to cobbles when non-colloidal | 4.0 | 0.030 |
| A-2 (A-2-4. A-2-5. A-2-6, A-2-7) | Mixture of GRAVEL and SAND, with silty or clay fines3, or nonplastic silt fines | Graded silts to cobbles when colloidal | 4.5 | 0.030 |
| same | same | Sandy loam, non-colloidal | 2.0 | 0.020 |
| A-3 | Fine SAND, without silty clay fines; e.g. beach sand or stream-deposited fine sand | Fine Sand, non-colloidal | 1.5 | 0.020 |
| same | same | Silt loam, non-colloidal | 2.3 | 0.020 |
| A-4 | Non- to moderately plastic4 SILT; mixtures of silt, sand, and/or gravel, with a minimum silt content of 36% | Alluvial silts, non-colloidal | 2.3 | 0.020 |
| A-5 | Moderately to highly plastic 4 SILT. Soil; mixtures of silt, sand, and/or gravel, with a minimum fines3 content of 36% | Ordinary firm loam | 2.5 | 0.020 |
| A-6 | Plastic4 CLAY soil; mixtures of clay, sand, and/or gravel, with a minimum fines3 content of 36% | Alluvial silts, colloidal | 3.5 | 0.025 |
| A-7 | Moderately to highly plastic, CLAY; mixtures of clay, sand, and/or gravel, with a minimum clay content of 36% | Stiff clay, very colloidal | 4.0 | 0.025 |

1)   Well-graded-containing a broad range of particle sizes with no intermediate sizes missing.

- Binder - soil particles consisting of fine sand, silt, and clay.

- Fines - particle sizes finer than 0.074 mm (e.g., silt and clay particles).

- Plasticity - ability of a soil mass to deform at constant volume without cracking or crumbling.

+    Relationship between AASHTO classification and Fortier and Scobey description is loosely correlated.