## Appendix 8D-1	Recommended Manning’s n-Values

| Type of Conduit |  | Wall Description | Manning’s n |
| --- | --- | --- | --- |
| Concrete Pipe |  | Smooth walls | 0.010-0.013 |
| Concrete Boxes |  | Smooth walls | 0.12-0.015 |
| Corrugated Metal |  | 2 2/3 by 1/2 inch corrugations | 0.022-0.027 |
| Pipes and Boxes |  |  |  |
| Annular or Helical |  | 6 by 1 inch corrugations | 0.022-0.025 |
| Pipe (n varies |  |  |  |
| Barrel size) |  | 5 by 1 inch corrugations | 0.25-0.026 |
| See HDS5 |  |  |  |
|  |  | 3 by 1 inch corrugations | 0.027-0.028 |
|  |  | 6 by 2 inch structural plate | 0.033-0.035 |
|  |  | 9 by 2 1/2 inch structural plate | 0.033-0.037 |
| Corrugated Metal |  | 2 2/3 by 1/2 inch corrugations | 0.12-0.024 |
| Pipes, Helical |  |  |  |
| Corrugations, |  |  |  |
| Full Circular Flow |  |  |  |
| Spiral Rib Metal |  | Smooth walls | 0.012-0.013 |
| *Note 1: | The Values indicated in this table are recommended Manning’s “n” design values. Actual Field values for older existing pipelines may vary depending on the effects of abrasion, corrosion, deflection and joint conditions. Concrete pipe with poor joints and deteriorated walls may have “n” values of 0.014 to 0.018. Corrugated metal pipe with joint and wall problems may also have higher “n” values, and in addition, may experience shape changes which could adversely effect the general hydraulic characteristics of the culvert. |  |  |
| Note 2: | For further information concerning Manning n values for selected conduits consult Hydraulic Design of Highway Culverts, Federal Highway Administration, HDS No. 5, page 163. |  |  |

Source:	HDS-5