Definitions:

Brim Drawdown Time		The time required for the entire calculated volume to drain from the basin.

Detention Basin		A stormwater management facility which temporarily impounds runoff and discharges it through a hydraulic outlet structure to a downstream conveyance system.  Since an extended detention basin impounds runoff only temporarily, it is normally dry during non-rainfall periods.

Extended Detention Basin		A stormwater management facility which temporarily impounds runoff and discharges it through a hydraulic outlet structure over a specified period of time to a downstream conveyance system for the purpose of water quality enhancement or stream channel erosion control.  Since an extended detention basin impounds runoff only temporarily, it is normally dry during non-rainfall periods.

Land Development Project	A manmade change to the land surface that potentially changes its runoff characteristics as a permanent condition.  The permanent condition should consider the effects of mature vegetative cover and should not be concerned with temporary changes due to construction activities.  Temporary changes are addressed by the VESCR.

Offsite		Refers to drainage area that is not part of the project construction.  At times, drainage area beyond the construction limits contributes to project discharges.  More often, the term is used when developing a stormwater management plan, and offsite impervious area is used to compensate for untreated project impervious area.

Q1 Control		This stormwater management measure is applied to channels with known or anticipated erosion problems as a quantity control measure.  In design, the entire contributing drainage area to the proposed basin is captured and used to develop the detention volume for a 1-year storm.

Abbreviations:

AASHTO	American Association of State Highway Transportation Officials

BMP	Best Management Practice

DCR	Department of Conservation and Recreation

HYG	Hydrograph

LDP	Land Development Project

MS	Minimum Standard

NRCS	National Resource Conservation Service, formerly Soil Conservation Service (SCS)

SCS	Soil Conservation Service

SWMR	Stormwater Management Regulations

VDOT	Virginia Department of Transportation

VESCR	Virginia Erosion and Sediment Control Regulations

WQV	Water Quality Volume