## Appendix 12C-2     	     Tidal Bridge Scour Data and Worksheet

### VIRGINIA DEPARTMENT OF TRANSPORTATION

#### TIDAL BRIDGE SCOUR DATA & WORKSHEET

Hydraulic Engineer:_________

Date:_________

##### BRIDGE LOCATION

BRIDGE No.	_________		Route:	_________	County No.	_________

Length:	_________ Ft.	River:	_____________________

TIDAL BRIDGE CATEGORY:	Islands		Semi-Enclosed		Estuary

Bays & Inlets

II.	CHANNEL CROSS SECTION

Channel Width (U/S 100 ft)	Wu = ______Ft.	Channel Width (at Bridge)	Wo 		______Ft.

Width (between abutment)	Wd = ______Ft.

Average Water Depth (below MSL/MLW/MTL)		D= _________Ft.

Clearance (from MSL/MLW/MTL to Lower Chord)	C= _________Ft.

Note:  Mean sea level (MSL), mean low water (MLW), mean tide level (MTL)

Skew Angle (Centerline of Bridge with Channel)	Phi () = _________ (Degrees)

- DRAINAGE AREA CHARACTERISTICS

(Information per USGS Report 94-4148 for Virginia Department of Transportation dated 1995)

Drainage Area: ___Sq. Mi.; Forest: F = ____%;	Average basin elevation: EL=_____Ft.

Main Channel Slope: SI=_____Ft/Mi;		Main Channel length:	L=_____Mi.

Peak Discharge Region Used:______________________________

Compute from USGS Regression Equation:

Qr100 = _________________CFS;			Qr500 = 1.7 (Qr100) = _________________CFS

- STORM TIDES

100-year		High Tide:	H100 = _________Ft.	Period:	T100 = _________Hrs.

500-year		High Tide:	H500 = _________Ft.	Period:	T500 = _________Hrs.

Surface Area of Tidal basin at		MSL:	As = _________Sq. Mi.

at _____Ft.:	As = _________Sq. Mi.

at _____Ft.:	As = _________Sq. Mi.

Compute Tidal Flows:

Qt100 = _________________CFS;			Qt500 = 1.7 (Qr100) = _________________CFS

- FLOW VELOCITY

  - Based on Cross Sectional Area at MSL/MLW

Cross Sectional Area, A1 = WoD = _________________Ft2

Vr100 = Qr100/A1 = _________Ft/S		Vr500 = Qr500/A1 = _________Ft/S

b)	Based on Cross Sectional Area at Midtide Elevation

Vt100 = (Qt100 + Q t100) / (A1 + WoH100/2) = _________	Ft/S

Vt500 = (Qt500 + Q t500) / (A1 + WoH500/2) = 			Ft/S

c)	Based on Manning Equation (n = 0.025; s = 0.0005)

Vt100 = 1.2 ((Qr100 = Qt100) / Wo)0.4 = _________	Ft/S

Vt500 = 1.2 ((Qr500 = Qt500) / Wo)0.4 = _________	Ft/S

Attach a Sketch of Cross-Section at Upstream (U/S) Side of Bridge