## Appendix 13B-1	Tidal Data and Conversions

#### National Geodetic Vertical Datum Above Mean Low Water

Locality				   			      NGVD-MLW (ft.)

Assateage beach Coast Guard Station, Chincoteague Isle			1.85

Chincoteague Island (north end of Chincoteague Channel)			0.77

Chincoteague Point, Chincoteague Bay					1.27

Wishart Point, Bogues Bay, Chincoteague Inlet				1.09

Wallops Island									1.86

Gargathy Neck								1.40

Oyster (off Sand Shoal Channel)						2.16

Kiptopeke Beach, Chesapeake Bay						1.42

Cape Charles Harbor, Chesapeake Bay					1.22

Gaskins Point, Occohannock river						0.94

Nandau Creek, Chesapeake Bay 						0.93

Harborton									0.76

Onancock, Onancock Creek, Chesapeake Bay				0.77

Lewisette, Potomac River							0.48

Dahlgren, Potomac River							0.55

Aquia Creek, Potomac River							0.35

Quantico, Potomac River							0.55

Alexandria, Potomac River							0.87

Washington, D.C.								0.91

Glebe Point Highway Bridge							0.48

Windmill Point, Rappahannock River						0.59

Mill Creek, Rappahannock River						0.62

Tappahannock, Rappahannock River						0.63

Saunders Wharf, Tappahannock River					0.39

Massapaponax, Sand and Gravel,

Fredericksburg, Rappahannock River						0.63

Gloucester Point, York River							1.25

West Point, York River							1.42

Old Point Comfort								1.30

Newport News, James River Entrance					1.31

Huntington Park, James River							1.39

Chuckatuck Creek Entrance							1.52

Menchville, James River							1.38

Burwell Bay, James River							1.17

Claremont, James River							0.78

Ferry Point, Chickahominy River						0.81

Lanexa, Chickahominy River							0.83

Norfolk Naval Shipyard, Portsmouth						1.44

Berkley, Norfolk Harbor							1.53

Sewells Point, Hampton Roads						1.38

Willoughby Spit, Chesapeake Bay						1.26

Little Creek Entrance, Chesapeake Bay					1.46

Little Creek									1.38

Virginia Beach									1.82