## Appendix 6D-2	Kinematic Wave Formulation

## Overland Flow

Comments:

VDOT has determined that the Kinematic Wave Method should only be used for:

a) Impervious Surfaces

b) n = 0.05 or less

c) Length = 300’ Maximum

d) See page 2 of 2 for suggested Manning’s roughness coefficients

## Appendix 6D-2	    Mannings Roughness Coefficient for Shallow Sheet Flow

Surface Description										n1

Smooth surfaces - concrete, asphalt, gravel, or bare soil (compacted)	0.011

Fallow – no residue (non-compacted bare, plowed soil)			0.05

Cultivated soils:

Residue cover < 20%									0.06

Residue cover > 20%									0.17

Grasses:

Short grass prairie										0.15

Dense grasses2											0.24

Bermuda grass											0.41

Range (natural)											0.13

Woods:3

Light underbrush											0.40

Dense underbrush										0.80

Soil Conservation Service Urban Hydrology for small water sheds Technical Release No. 55, Natural Resources Conservation Service, Washington, D.C. 1986

1	The n values are a composite of information complied by Engman (1986).

2	Includes species such as weeping lovegrass, bluegrass, buffalo grass, blue grama grass and native grass mixtures.

3	When selecting n, consider cover to a height of about 1 inch.  This is the only part of the plant cover that will obstruct sheet flow.

Source: AASHTO 2005 MODEL DRAINAGE MANUAL  (text shown in parentheses are VDOT additions to the original chart which were included to simplify interpretation and application)