## Appendix 6E-1	Rational Method Runoff Coefficients

## Recommended Coefficient of Runoff Values for Various Selected Land Uses

| #### Description of Area | Runoff Coefficients |
| --- | --- |
| Business: Industrial and Commercial | 0.80-0.90 |
| Apartments and Townhomes | 0.65-0.75 |
| Schools | 0.50-0.60 |
| Residential - lots 10,000 sq. ft. | 0.40-0.50 |
| - lots 12,000 sq. ft. | 0.40-0.45 |
| - lots 17,000 sq. ft. | 0.35-0.45 |
| - lots ½ acre or more | 0.30-0.40 |
| Parks, Cemeteries and Unimproved Areas | 0.20-0.35 |
| Paved and Roof Areas | 0.90 |
| Cultivated Areas | 0.50-0.70 |
| Pasture | 0.35-0.45 |
| Lawns | 0.25-0.35 |
| Forest | 0.20-0.30 |
| Steep Grass (2:1)* | 0.40-0.70 |
| Shoulder and Ditch Areas * | 0.35-0.50 |

Comments:

1.	The lowest range of runoff coefficients may be used for flat areas (areas where the majority of the grades and slopes are 2% and less).

2.	The average range of runoff coefficients should be used for intermediate areas (areas where the majority of the grades and slopes are from 2% to 6%).

3.	The highest range of runoff coefficients shall be used for steep areas (areas where the majority of the grades are greater than 6%), for cluster areas, and for development in clay soil areas.

4.	See Appendixes 6E-2, 6E-3, 6E-4 and 6E-5 for runoff coefficients with the Cf factor applied.

*Lower runoff coefficients should be used for permanent or established conditions (post- construction), i.e. sizing stormwater management basins.

*Higher runoff coefficients should be used to design roadside ditch linings (construction). The design considers the ditch lining as not yet established.

Comments: Runoff Coefficients compiled from various sources.