## Appendix 7B-5			RIPRAP DESIGN WORK SHEET

## For other than VDOT Standard Riprap Sizes

CHANNEL DATA

Q = _________(cfs) 		P = _________(ft.) 		n = ________

So = _________(ft/ft)     	R = _________(ft.)

dn = _________(ft.)      	Vn = _________(fps)

A = _________(ft2)   		Side Slope = _____ :1

ASSUMED RIPRAP SIZE -  D50 = ________

#### VERIFY ASSUMED RIPRAP SIZE

 = _______o  (Appendix 7E-1)

Side Slope = _____ : 1     = ______o

K1 = [1 - (sin2  / sin2 )] 0.5

K1 = [1 - (sin2 _______o / sin2 _______o)] 0.5 = ________

For Specific Gravity  = 2.65 and Stability Factor = 1.2

D50 = 0.001  Va3 / (davg 0.5  K1 1.5)

D50 = 0.001  ________3 / ( ________0.5  ________1.5) =  _____________

D50 Computed ( ______ )  (<) (=) (>)  D50 Assumed ( ______ )

Assumed D50 is  (correct)  (incorrect)

Note: The above process of assuming a D50 size, determining the natural angle of repose () and computing a D50 size should be repeated until the Assumed D50 size equals the Computed D50 size. Once the D50 size determination has been made, it should be adjusted for the Specific Gravity Correction Factor Csg (if any) and the Stability Correction Factor (CSF) (if any) to derive a Final D50.

Correction Factor For Riprap Specific Gravity (Ss) other than 2.65 (Default = 1.0)

Csg = 2.12 / (Ss - 1)1.5  =  2.12 / ( ________ - 1)1.5  =  _________

Correction Factor For Stability Factor (SF) other than 1.2 (Default = 1.0)

CSF = (SF / 1.2)1.5 = ( ________ / 1.2)1.5 = ________

Final Correction Factor = C = Csg  CSF = ________  ________ = ________

Final D50 = C  Computed D50 = ________  ________ = ________

RIPRAP RECOMMENDATION:  _________________

Thickness (T ) = _________” (2  D50 MSD minimum)

Comments and/or Source

Source:	VDOT

Comments and/or Source