## Appendix 7D-5	Selection of Stability Factors

| CONDITION | STABILITY FACTOR RANGE |
| --- | --- |
| Uniform flow; straight or mildly curving reach (curve radius/ channel width >30); impact from wave action and floating debris is minimal; little or no uncertainty in design parameters. Gradually varying flow; moderate bend curvature (30 > curve radius/channel width > 10); impact from waves or floating debris is moderate. Approaching rapidly varying flow; sharp bend curvature (30 > curve radius/channel width >10); significant impact potential from floating debris and/or ice; significant wind and/or bore generated wves (1-2 ft); high flow turbulence; mixing flow at bridge abutments; significant uncertainty in design parameters. Channel bends when ratio of curve radius to channel width (R/W) > 30. Channel bends when 30 > R/W > 10. Channel bends when R/W < 10. | - - 1.2 1.3 - 1.6 1.6 - 2.0 1.2 1.3 - 1.6 1.7 |