## Appendix 8D-2	Entrance Loss Coefficients (Ke),

## Outlet Control, Full or Partly Full

| Type of Structure and Design of Entrance Coefficient |  |  |
| --- | --- | --- |
| # Pipe, Concrete |  |  |
| Mitered to conform to fill slope 0.7 |  |  |
| *End-Section conforming to fill slope 0.5 |  |  |
| Projecting from fill, sq. cut end 0.5 |  |  |
| Headwall or headwall and wingwalls Square-edge 0.5 |  |  |
| Rounded (radius = D/12) 0.2 |  |  |
| Socket end of pipe (groove-end) 0.2 |  |  |
| Projecting from fill, socket end (groove-end) 0.2 |  |  |
| Beveled edges, 33.7 or 45 bevels 0.2 |  |  |
| Side-or slope-tapered inlet 0.2 |  |  |
| # Pipe, or Pipe-Arch, Corrugated Metal |  |  |
| Projecting from fill (no headwall) 0.9 |  |  |
| Mitered to conform to fill slope, paved or unpaved slope 0.7 |  |  |
| Headwall or headwall and wingwalls square-edge 0.5 |  |  |
| *End-Section conforming and to fill slope 0.5 |  |  |
| Beveled edges, 33.7 or 45 bevels 0.2 |  |  |
| Side-or slope-tapered inlet 0.2 |  |  |
| # Box, Reinforced Concrete Headwall parallel to embankment (no wingwalls) Square-edged on 3 edges 0.5 Rounded on 3 edges to radius of D/12 or B/12 or beveled edges on 3 sides 0.2 |  |  |
| Wingwalls parallel (extension of sides) |  |  |
| Square-edged at crown 0.7 |  |  |
| Wingwalls at 10 to 25 to barrel |  |  |
| Square-edged at crown 0.5 |  |  |
| Wingwalls at 30 to 75 to barrel |  |  |
| Crown edge rounded to radius of D/12 or beveled top edge 0.2 |  |  |
| Square Edge at crown 0.4 |  |  |
| Side-or slope-tapered inlet 0.2 |  |  |
| *Note : | “End Sections conforming to fill slope,” made of either metal or concrete, are the sections commonly available form manufacturers. From limited hydraulic tests they are equivalent in operation to a headwall in both inlet and oulet control. Some end sections, incorporating a closed taper in their design have a superior hydraulic performance. These latter sections can be designed using the information given for the beveled inlet. |  |

Source HDS-5