- TYPE OF BMP-QUALITY CONTROL – Determine the type of BMP to be used from Table 11-1.  Find the new percent impervious area within the project area (right of way and permanent easements) per outfall.

- WATER QUANTITY CONTROL – Check for an adequate receiving channel in accordance with MS-19 of the erosion and sediment control regulations.  If the receiving channel is not adequate, the BMP must provide attenuation of the post-development peak discharge to pre-development discharge levels.

- Natural Channels:	Q2 for discharge and velocity

- Man-made Channels:  Q2 for velocity and Q10 for discharge

- Storm Drainage Systems:	Q10 for capacity

- WATER QUANTITY CONTROL (ALTERNATIVE) – Control of the runoff from the 1-year frequency storm, in lieu of the 2- and 10-year frequency storms, may be required if:

- A field survey of the receiving channel indicates that significant erosion is occurring under existing conditions

- It is anticipated that erosion may occur in the receiving channel due to increased frequency of bankful flow conditions as a result of standard peak flow attenuation

If attenuation of the 1-year frequency storm is required, the volume requirements are based upon containing the entire volume of runoff from the 1-year frequency event for a period of 24-hours.

- WATER QUALITY CONTROL – Determine the required water quality volume (WQV) using Table 11-1 and compute the volume requirements.

- TEMPORARY SEDIMENT STORAGE – If the BMP is to be used as a temporary sediment basin during construction, calculate the volume requirements:

- Wet Storage – 67 cu. yds. per acre of the total contributing drainage area plus

- Dry Storage – 67 cu. yds. per acre of the total contributing drainage area

- FOREBAY – If the BMP is to have a sediment/debris forebay, calculate the volume requirements.  Forebays are recommended for most types of basins.

7.	OTHER DESIGN CONSIDERATIONS

- Use “Design Guidelines for SWM Basins”

- Use “Details for Design of Dams”

- Use “Perimeter Control Guidelines”

- Design of the emergency spillway for conveying Q100

- Request foundation information for basin and dam

- Request aquatic planting plan from the Environmental Division (when required)

- Provide maintenance access with turnaround (include chain barricade when required)

- Provide sufficient right-of-way and easement for construction and maintenance

- Provide information for Stormwater Management Data Base (complete the “SWM Facility – Tabulation Sheet” provided in Appendix 11E-1)