# Sign Designer / reviewer checklist

The following checklist provides both the sign designer and the TE/L&D Manager guidelines for preparing sign plans during the different phases of design.  The checklist is designed to correlate with the Concurrent Engineering Process discussed in the TEDM Section I – General, Chapter 2.

Project Name:

Project No: 	UPC / PPMS No:

TE/L&D Manager:

Sign Designer:

Advertisement Date:

## scoping Team Meeting					Date of Meeting: _________

- The following actions should take place prior to or during the Scoping Team Meeting.

- 1)	Ensure that adequate space for signing has been incorporated into the conceptual roadway typical section(s).

- 2)	Ensure that the limits of survey will accommodate the signing design.

- 3)	Identify the potential for overhead sign structures.

- 4)	Identify other design elements that require early coordination with the sign design, (roadway lighting, traffic signals, pavement markings, etc.).

- 5)	Determine the agency and contact person that will own and maintain the signs.

- 6)	Determine the agency and contact person that will be responsible for sign design and plans.

- 7)	Prepare project budget estimate.

Scoping review completed by:

Dated reviewed:

## Preliminary Field Inspection Team Meeting 	Date of Meeting: _________

- The following actions should take place prior to or during the Preliminary Field Inspection Team Meeting.

- 1)	Send the Sign Design Questionnaire to the agency responsible for maintaining signs (District, Residency, Municipality, etc). [Appendix IIB-1]

Sent To:		Date Sent:

- 2)	Evaluate the roadway geometry and typical sections to ensure that appropriate right of way and (or) easements have been provided for signs and sign pole installation.

- 3)	Update project budget estimate.

Pre-F.I. review completed by:

Dated reviewed:

## Public Hearing Team Meeting				Date of Meeting: _________

- The following actions should take place prior to or during the Public Hearing Team Meeting.

- 1)	Perform a sign inventory to identify existing signs and structures.

- 2)	Develop sign base plans.

- 3)	Develop conceptual sign plans showing major overhead and guide sign messages and locations.

- 4)	Coordinate with District for sign messages and placement of guide signs.

- 5)	Determine if any overhead structures will require lighting.

- 6)	Determine availability of electrical power if overhead structures are to be lighted. [Appendix VB-3]

- 7)	Verify right of way and (or) easements are provided for proposed sign infrastructure.

- 8)	Coordinate sign design with structure and bridge designer when signs are to be located on bridges.

- 9)	Coordinate sign design with landscape, utility, median barriers, retaining walls, noise walls, etc.

- 10)	Ensure coordination of all TCD plans.

- 11)	Update project budget estimate.

Public Hearing review completed by:

Dated reviewed:

## Field Inspection Team Meeting				Date of Meeting: _________

- The following actions should take place prior to or during the Field Inspection Team Meeting.

### Sign Plan General Overview

- 1)	Address public hearing comments.

- 2)	Update conceptual sign design plans with current roadway & utility design plans.

### Sign Plan Coordination Issues

- 1)	Verify signs do not conflict with overhead or underground utilities.

- 2)	Coordinate sign plans with Highway Signs Programs Section of Traffic Engineering Division.

- 3)	Review sign locations for compatibility with noise walls, median barriers, retaining walls, landscaping, etc.

- 4)	Verify the proposed or existing grade is accessible for maintenance and suitable for sign installation.

- 5)	Coordinate electrical service items for those sign structures requiring sign lighting.

### Construction Cost Estimate

- 1)	Update project budget estimate.

F.I. review completed by:

Dated reviewed:

## 80% Plan Submittal					Date of Submission: ___________

- Advance to preliminary sign design plans for review by Maintaining Jurisdiction, Residency, District Traffic and Utility Sections and Central Office. Plans are approximately 80% complete such that the reviewer can provide guidance.

80% DESIGN ELEMENTS

### Sign Plan Set General Overview

Stand alone Sign Plans

  - 1)	Provide Title Sheet - includes latest design standard and specifications.

  - 2)	Provide Location Map.

  - 3)	Provide Revision Data Sheet.

  - 4)	Provide Survey & Alignment Data Sheets, if applicable.

  - 5)	Provide Maintenance of Traffic Sheets, if applicable.

Requirements for all Sign Plans

- 1)	Use Sheet Cell from the VDOT Cell Libraries.

- 2)	Incorporate design guidance provided in Sign Questionnaire. [Appendix IIB-1]

- 3)	Provide Index of Sheets.

- 4)	Provide Insertable Sheets.

- 5)	Provide Sign Schedule Sheets.

- 6)	Provide Sign Detail Sheets (Figure sheets, Elevation sheets, etc.).

- 7)	Provide Notes for Safety Improvement Sheet. [Appendix IIA-9]

- 8)	Show Project Data on plan sheets, (Project No., Series and Page No., etc.).

- 9)	Show Scale, North Arrow, Match Lines, etc.

### Sign Plan Coordination Issues

Roadway Plans

- 1)	Ensure that poles, conduit and junction boxes are not in conflict with drainage structures, retaining wall tiebacks, noise wall footers, etc. [Sec. V, Chapter 3, 3.4]

- 2)	Evaluate proposed or existing grades for suitable installation of the pole foundations and trenching of conduit. [Sec. V, Chapter 3, 3.4]

- 3)	Reference associated roadway plan sheets in the reference box on sign plan sheet, (Series No. and Page No.). [Appendix IIA-3]

Pavement Marking/Marker Plans

- 1)	Coordinate sign placement as they relate to pavement markings (e.g. stop bars, lane drops or exit only lanes, transitioning out lanes, railroad crossings, and school zones).

- 2)	Reference associated pavement marking/marker plan sheets in the reference box on sign plan sheet, (Series No. and Page No.). [Appendix IIA-3]

## 80% Plan Submittal (Continued)

Traffic Signal Plans

- 1)	Coordinate sign layout as they relate to the traffic signal infrastructure and design.

- 2)	Include proposed signs mounted on signal mast arms and poles in the sign schedule.

- 3)	Reference associated traffic signal plan sheets on the sign plan sheet, (Series No. and Page No.). [Appendix IIA-3]

Lighting, Utilities & Bridges Plans

- 1)	Review sign locations on bridges with the structure and bridge designer.

- 2)	Identify power source for sign lighting and show electrical service connection.

- 3)	Coordinate power requirements and electrical distribution for the sign lighting with lighting designer.

- 4)	Verify power provider approves sign electrical service connection location.

- 5)	Ensure sign infrastructure (overhead sign structures, foundations, etc.) are not in conflict with overhead or underground utilities.

- 6)	Reference associated lighting, utility and bridge plan sheets in the reference box on sign plan sheet, (Series No. and Page No.). [Appendix IIA-3]

### Sign Plan Layout

Existing Sign Information

- 1)	Show existing sign structures and messages at existing sign locations. (Signs are to be delineated with dash lines). [Appendix IIA-3]

- 2)	Indicate type of sign or structure in top of ellipse symbol.

- 3)	Indicate action for measurement / payment in bottom of ellipse symbol.

- 4)	Orient sign messages on plan sheet to read in direction of travel.

Proposed Sign Information

- 1)	Update sign plan base sheets with most current roadway design plan sheets.

- 2)	Ensure all sign structures are located within right of way and (or) easements.

- 3)	Show proposed sign structures and messages at proposed locations. (Signs are to be delineated with solid lines). [Appendix IIA-3]

- 4)	Indicate sign number in top of circle symbol.

- 5)	Indicate text number in bottom of circle symbol.

- 6)	Orient sign messages on plan sheet to read in direction of travel.

- 7)	Identify geometric, utility, retaining walls, guardrail and other possible conflicts for resolution with other design disciplines.

- 8)	Locate all overhead sign structures outside of the clear zone or protect sign structures with guardrail or barriers.

- 9)	Ensure plan sheet items match legend and notes are applicable.

## 80% Plan Submittal (Continued)

Proposed Sign Information for Sign Lighting and Associated Electrical Service

- 1)	Ensure provisions are made for a sign lighting retrieval system.

- 2)	Ensure junction boxes and conduits are proper size, type, and located within right of way when providing sign lighting and project does not include roadway lighting.

- 3)	Provide electrical grounding conductors in all non-metallic conduits.

- 4)	Identify electrical service type for overhead sign structures on plan sheet and ensure it matches Summary of Quantities pay item.

### Sign Detail Sheets

- 1)	Properly label Figure sheets with correct message, dimensions, text size and style indicated.

- 2)	Provide a sign structure detail sheet, if required. [Appendix IIA-11]

- 3)	Provide overhead sign structure elevation detail sheet, if required. [Appendix IIA-8]

- 4)	Provide notes for safety improvement items sheet. Measurement and payment items not used are to be marked out with an “X” or deleted from sheet.

- 5)	Provide “EXIT PANEL MOUNTING DETAIL SHEET”, if applicable.

### Sign Schedule Sheet

- 1)	Arrange order of signs in schedule in accordance with the priority given in the MUTCD (e.g. first –regulatory signs, second – warning signs, third – guide signs, etc.), whenever possible.

- 2)	Cross-check all labeling information in the schedule with plan sheets. Special information for panels and structures are noted in the remarks column.

- 3)	Verify all information indicated on the sign schedule is correct.

### Construction Cost Estimate

- 1)	Show all Standard and Non-Standard Pay Items in the Summary of Quantities sheet.

- 2)	Unit quantities do not need to be shown on Summary of Quantities sheet for this submittal.

- 3)	Prepare construction cost estimate.

## 80% Plan Submittal (Continued)

Submit Plan To: (If Applicable)	Contact Person	Date Sent

Municipality

VDOT, Residency

VDOT, District Traffic Engineering

VDOT, District Utilities Section

VDOT, CO – Traffic Engineering (L&D)

VDOT, CO – Traffic Engineering Division

VDOT, Scheduling and Construction

FHWA

80% review completed by:

Dated reviewed:

## QC Plan Submittal					Date of Submission: ___________

- Advance the 80% sign design plans to 100% design completion.

### Respond / Incorporate Review Comments From:

- 1)	Municipality (if applicable)

- 2)	VDOT, Residency

- 3)	VDOT, District Traffic Engineering

- 4)	VDOT, District Utilities Section

- 5)	VDOT, CO – Traffic Engineering (L&D)

- 6)	VDOT, CO – Traffic Engineering Division

- 7)	VDOT, Scheduling and Construction

- 8)	FHWA

## Constructability Check

- 1)	Ensure clear zone requirements are met.

- 2)	Verify sign structures are breakaway or structures are outside of the deflection clearance of guardrails.

- 3)	Update plans with other design disciplines to ensure that conflicts with drainage, utilities (overhead or underground) and landscaping have not been created with design revisions.

- 4)	Verify right of way and (or) easements are sufficient to install sign infrastructure.

### Plan Sheets

- 1)	Finalize General Notes.

- 2)	Ensure plan sheet call-outs are consistent with legend and Summary of Quantities pay items, and Transport pay items.

- 3)	Verify each pay item is clearly identified.

- 4)	Perform quantity take-offs and verify each pay item in the project is accounted for in the Summary of Quantities.

### Plan Detail Sheets

- 1)	Finalize Sign Panel Detail Sheets.

- 2)	Finalize (Figure Sheet) Elevation Detail Sheets.

- 3)	Finalize Special Plan Details (Median barrier mounting details, special sign mountings, etc.).

- 4)	Include appropriate Insertable Sheets.

- 5) 	Finalize Notes for Safety Improvement Items Sheet.

## QC PLAN SUBMITTAL (CONTINUED)

### Summary of Quantities / Special Provisions

- 1)	Finalize Summary of Quantities Sheet. [Appendix IIA-12], [Appendix IIA-13]

- 2)	Verify measurement and payment for each pay item and ensure it matches the VDOT standard items or non-standard items as discussed in the Specifications, Special Provisions and/or Special Provision Copied Notes. [Sec. I, Chapter 3, 3.8]

- 3)	Discuss in detail the Special Provisions and Special Provision Copied Notes with Traffic Engineering Division Specifications Section.

### Construction Cost Estimate

- 1)	Verify TRNS·PORT quantities match Summary of Quantities sheet.

- 2)	Develop costs estimates per unit price for non-standard pay items and review with TE/L&D Manager.

### Plan Set Submitted (as necessary)

- 1)	Title Sheet (Stand alone project)

- 2)	Location Map Sheet (Stand alone project)

- 3)	Index of Sheets

- 4)	Revision Data Sheets (Stand alone project)

- 5)	Survey & Alignment Sheets (Stand alone project)

- 6)	Maintenance of Traffic Sheets (Stand alone project)

- 7)	General Notes (Stand alone project)

- 8)	Insertable Sheets

- 9)	Notes for Safety Improvement Items Sheet

- 10)	Summary of Quantities Sheet

- 11)	Sign Schedule Sheets

- 12)	Non Standard Post and Ground Mount Structure Detail Sheet

- 13)	Exit Panel Mounting Detail Sheets

- 14)	Legend and Detail Sheets

- 15)	Plan Sheets

- 16)	Plan Detail Sheets

### Special Provisions Submitted

- 1)	Special Provisions:

- 2)	Non-Standard Items:

### Up-Load Plans to Falcon in TIF format. 		Date complete: __________________

- 1)	Create Plan Index in Excel format.

QA review completed by:

Dated reviewed:

## Pre-Advertisement Conference		Date of meeting: ______________

- Advancing 100% design plans and specifications to construction plan set.

### Respond / Incorporate Review Comments From:

- 1)	Municipality (if applicable)

- 2)	VDOT, Residency

- 3)	VDOT, District Traffic Engineering

- 4)	VDOT, District Utilities Section

- 5)	VDOT, CO – Traffic Engineering (L&D)

- 6)	VDOT, CO – Traffic Engineering Division

- 7)	VDOT, Scheduling and Construction

- 8)	FHWA

### Update Plans on Falcon. 			Date complete: ________________________

### Up-Load Plans to Falcon in TIF format. 	Date complete: ________________________

- 1)	Update Plan Index.

### Revise Special Provisions. 			Date complete: ________________________

### Update TRNS-PORT Cost Estimate.	Date complete: ________________________

- 1)	Verify updated TRNS·PORT quantities match Summary of Quantities sheet.

PAC review completed by:

Dated reviewed: