Separate Left Turn Phase Warrant

“Kentucky Method”

Project No. __________________________

Intersection: ______________________________________________________

The traffic volumes used below in this analysis are for peak hour volumes.  The analysis should consider peak hour volumes + 5 year traffic volume projections for design of new traffic signals.

Analysis:

Volume of Left Turn Vehicles  X  Volume of Opposing Through Vehicles =  “X”

Number of Opposing Through Lanes

“X” = Left Turn Conflict Factor

1.	If “X” is less than 50,000, a separate left turn phase may not be required.

2.	If “X” is equal to or greater than 50,000, a separate left turn phase should be considered.

Example Problem

NB	100 X 1150  =  57,500			Greater than 50,000

2

SB	 50  X  450   =  11,250			Less than 50,000

2

WB	 50  X  650   =  16,250			Less than 50,000

2

EB	 50  X  650   =  16,250			Less than 50,000

2

Northbound left turn meets warrants for a separate left turn phase.