# PRELIMINARY PLANS

Note:	These directions are for the electronic handling of a preliminary set of plans and assume the preliminary plan sheets are in Falcon, have been reviewed and are ready for distribution. Preliminary plans should be complete on or before the Field Inspection stage shown on the Electronic Plan Submission summary chart.

| # | ## Description |
| --- | --- |
| 1 | Index the sheet in Falcon, inserting the document number (use 01, 02 etc. for this stage), plan number, sheet description, and choose current drawings from the Generate Tif block under File Format Keywords. If the plan number has not been assigned, contact the Central Office S&B file room to obtain one. |
| 2 | Create .tif files utilizing the Bridge Tif File Generator configuration. - Fence file for plotting in normal manner. - Active CFG in the PRF Generator window must be changed to Bridge Tif File Generator. English unit projects should require no other changes. Metric unit projects will require the scale to be changed as if plotting. This creates only an electronic .tif file. No hardcopy plot will be created. |
| 3 | Transmit the preliminary distribution form electronically - Upon receipt, the generated .tif files will be renamed to XXX-XXx_YY. - XXX-XXx: Bridge Plan Number (little x is A, B etc.) - YY: Sheet Number (may be YYY if over 99 sheets). NOTE: YYY format should be used for ALL sheets if the total is expected to be greater than 99. - Renamed .tif files for projects can be viewed in the Falcon electronic Plan File Room environment under Bridge Plans\district\Structure plan no. |
| 4 | Notify the Central Office Computer Operations section, using email, the project is ready to be placed in the Bridge Plan File Room as a preliminary. John Halsey, Jr.: John.Halsey@VirginiaDOT.org 804-786-9189, or Doug Horton: Douglas.Horton@VirginiaDOT.org 804-786-1315 A folder will be generated in the Bridge Plan File Room, the preliminary tif files will be added, and the designer notified upon completion. The preliminary plan assembly will also be made available through the Falcon Web Current Drawings. Preliminary plans are the only stage that will be made available through Current Drawings at this time. |
| 5 | Upon notification that the plans are available in the Plan File Room, index the .tif files, adding the plan number, project number and description as a minimum. |