TIPS FOR PREPARING CONSTRUCTION/RIGHT OF WAY TITLE SHEET FOR APPROVAL SIGNATURES

- The project limits, description and Federal Aid Numbers should match Integrated Project Manager (IPM)

- If Federal Funded, (NFO) or (FO) must be in the upper right corner before the Project Number – for example:  (FO) 0095-96A-101, C501

- Correct titles on the signature blocks:

Programming Division Director

State Location and Design Engineer

Chief Financial Officer

Chief Engineer 
Commissioner (RW title sheet only)

- If the project is an M (Minimum Plan) with a bridge number, the tabulation block needs to be shown at the bottom of the title sheet with descriptions for each job number (If the project is an M only, the UPC number should be located at the top with the FHWA-534 Data Number)

- Traffic has been updated (not more than 2 years old)

- The tabulation block should always show the project base number along the left side

- The Construction project limits in the tabulation block and the Construction project limits in the center of the sheet should match

- If the project limits for RW are different from Construction, the RW project limits should be shown in the tabulation block

- The District name should be on the left border of the title sheet along with the supervisor/designer names and phone numbers

- If the title sheet is not designed by VDOT, please make sure the consultant PE seal is on the original mylar title sheet, with signature, license number and dated

- If RW title sheet, it must have the note “These plans are unfinished and are not to be used for any type of construction”

- If Construction title sheet, the above note must be removed

- If the proposed construction will be within the existing right of way, the note “All construction is to be performed within existing right of way” should be shown on the area adjacent to the RW approval signature block

- The consultant shall place a note in the .dgn file (with typed name, license number and the date signed) on the Title Sheet, or front sheet for bridge projects, etc.

Example:

Signed and sealed by: NAME OF SIGNER LICENSE NUMBER On the date of: SIGNATURE DATE A copy of the original signed and sealed sheet is available.

CONSULTANT LOGO

- Immediately upon signing, the original signed title sheet (right of way and construction stage) will be filed in the VDOT Central Office Plan Library

- The type written names of the signers, and date signed (i.e. John W. Doe, signed October 29, 2003), will be inserted into the signature blocks of the .dgn version of the Title Sheet

- Scanned signatures shall not be on the dgn file or the tiff file of the title sheet

- Locality Administered must have the city or county signatures on title sheet before submitting for VDOT signatures

(See IIM-LD-204 for more information)